|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # SCHOOL OF MECHANICAL ENGINEERING |
| --- | --- |

B

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MECHANICAL |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- The depth of water below the spillway and after hydraulic jump are 1 m and 6 m respectively. The head lost will be

(A) 1.04 m	(B) 5 m

(C) 1.7 m	(D) 2.05 m

- The motion of a fluid is vortex if each particle of the fluid moves in a circular path with the speed which

(A) is constant

(B) is directly proportional to distance from centre

(C) varies as square of distance from centre

(D) varies inversely as the distance from centre

- According to Avogadro's law, for a given pressure and temperature, each molecule of a gas

(A) occupies volume proportional to its molecular weight

(B) occupies volume proportional to its specific weight

(C) occupies volume inversely proportional to its molecular weight

(D) occupies same volume

- On volume basis, air contains following parts of oxygen
(A) 21
(B) 23
(C) 25
(D) 77

- A perfect gas at 27°C was heated until its volume was doubled. The temperature of the gas will now be
(A) 270°C
(B) 540°C
(C) 327°C
(D) 729°C

- A grey body is one whose absorptivity
(A) varies with temperature
(B) varies with wavelength of the incident ray
(C) is equal to its emissivity
(D) does not vary with temperature and wavelength of the incident ray

- If two surfaces of area A distance L apart, of a material having thermal conductivity k are at temperatures t1 and t2, then heat flow rate through it will be

(A) KA/L  x  (t1-t2)

(B) KL/A  x  (t1-t2)

(A) K/LA  x  (t1-t2)

(A) L/KA  x  (t1-t2)

- If the compression ratio of an engine working on Otto cycle is increased from 5 to 7, the %age increase in efficiency will be

(A) 2%

(B) 4%

(C) 8%

(D) 14%

- Big refrigeration plants (of capacity of the order of 1000 tons) usually employ following type of refrigeration plant

(A) vapour absorption

(B) vapour compression

(C) air cycle

(D) electrolux refrigerator

- Wet bulb temperature is
	(A) indication of amount of moisture in air
	(B) measured by wetting the bulb of the thermometer
	(C) less than dry-bulb temperature
	(D) dependent on the dryness and temperature of air

- The ratio of diameters of Low Pressure (P1) Cylinder to High Pressure  (P2) Cylinder  with complete intercooling is equal to

(A) P2/P1

(B) P1/P2

(C) SQRT OF P2/P1

(D) SQRT OF P1/P2

- Turbofan engine employs

(A) one air stream

(B) two or more air streams

(C) no air stream

(D) solid fuel firing

- Pick up the wrong statement about water tube boiler in comparison to fire tube boilers
	(A) former generates steam at high pressure
	(B) former occupies less space for same power
	(C) rate of steam flow is more in former case
	(D) chances of explosion are less in former case.

- Bomb calorimeter is used to determine

(A) Higher calorific value at constant volume

(B) Lower calorific value at constant volume

(C) Higher calorific value at constant pressure

(D) Lower calorific value at constant pressure

- Most serious energy loss in impulse steam turbine is

(A) steam leakage between diaphragms and shaft

(B) leakage over blade ends through radial clearance passages

(C) frictional resistance between steam and nozzle side

(D) impact loss as steam enters the moving blades

- The resultant of the following three couples

20 kg force, 0.5 m arm, + ve sense

30 kg force, 1 m arm, – ve sense

40 kg force, 0.25 m arm, + ve sense

having arm of 0.5 m will be

(A) 20 kg, – ve sense

(B) 20 kg, + ve sense

(C) 10 kg, + ve sense

(D) 10 kg, – ve sense

- Two bodies of 100 kg and 400 kg are resting on two inclined planes α and β towards each other and the bodies are joined together by a string passing over a pulley connected at the top of inclined planes. Coefficient of friction for two bodies with their inclined planes are μ₁ and μ₂. Tension in string will be

(A) 100 kg

(B) 300 kg

(C) 400 kg

(D) 500 kg

- To transmit power from one rotating shaft to another whose axes are neither parallel nor intersecting, use

(A) spur gear

(B) spiral gear

(C) bevel gear

(D) worm gear

- In the case of flat collar bearing, the frictional force in case of uniform pressure can be assumed to be acting at

(A)  2 /3  x  (R12   - 2 R22   / (R12– R22)

(B)  2 /3  x  (R13   - 2 R23   / (R12– R22)

(C)  1/2  x (R1 + R2)

(D) 2/3 x (R1 + R2)

- Which of the following is spring controlled governor?
	(A) Hartnell
	(B) Hartung
	(C) Wilson-hartnell
	(D) all of the above.

- When two springs are in series (having stiffness K), the equivalent stiffness will be

(A) K

(B) K/2

(C) 2K

(D) K/4

- The maximum efficiency of a screw jack having square threads and friction angle of 30° will be

(A) 11%

(B) 20%

(C) 30%

(D) 33%

- Two stainless steel foils of 0.1 mm thickness are to be joined. Which of the following processes would be best suited

(A) Gas welding

(B) Electroslag welding

(C) TIG welding

(D) MIG welding

- Which is incorrect statement about results of cold working?

(A) it increases corrosion resistance

(B) it increases strength, elasticity and hardness with a corresponding decrease in ductility

(C) handling of parts is easier

(D) dimension tolerances and finish are high

- Tool life is most affected by machine

(A) cutting speed

(B) tool geometry

(C) feed and depth

(D) microstructure of material being cut