|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # SCHOOL OF MECHANICAL ENGINEERING |
| --- | --- |

A

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MECHANICAL |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- A liquid jet from a nozzle exposed to atmosphere traverses along

(A) a straight line	(B) a circular path

(C) an elliptical path	(D) parabolic path

- The included angle for triangular notch for maximum discharge is

(A) 45o	(B) 60o

(C) 90o	(D) 108o

- The depth of centre of pressure in a rectangular lamina of height h with one side in the liquid surface is at

(A) h

(B)  h/3

(C) 2h/3

(D) h2

- If a certain amount of dry ice is mixed with same amount of water at 80°C, the final temperature of mixture will be

(A) 80o C

(B) 0o C

(C) 40o C

(D)  20o C

- 2 kg of substance receives 500 kJ and undergoes a temperature change from 100°C to 200°C. The average specific heat of substance during the process will be

(A) 5 KJ / Kg.K

(B) 2.5 KJ / Kg.K

(C) 10 KJ / Kg.K

(D) 25 KJ/Kg.K

- In convection heat transfer from hot flue gases to water tube, even though flow may be turbulent, a laminar flow region (boundary layer of film) exists close to the tube. The heat transfer through this film takes place by
(A) convection
(B) radiation
(C) conduction
(D) both convection and conduction

- Two balls of same material and finish have their diameters in the ratio of 2 : 1 and both are heated to same temperature and allowed to cool by radiation. Rate of cooling by big ball as compared to smaller one will be in the ratio of
(A) 1 : 1
(B) 2 : 1
(C) 1 : 2
(D) 4 : 1

- In current carrying conductors, if the radius of the conductor is less than the critical radius, the addition of electrical insulation is desirable, as

(A) it reduces the heat loss from the conductor and thereby enables the conductor to carry a higher current.
(B) it increases the heat loss from the conductor and thereby enables the conductor to carry a higher current.
(C) it increases the thermal resistance of the insulation and thereby enables the conductor to carry a higher current.
(D) it reduces the thermal resistance of the insulation and thereby enables the conductor to carry a higher current.

- The specific fuel consumption per BHP hour for a petrol engine is approximately

(A) 0.15 kg
	(B) 0.2 kg
	(C) 0.25 kg
	(D) 0.3 kg

- Which of the following refrigerants has highest refrigeration efficiency?

(A) Freon-11

(B) Freon-22

(C) R-12

(D) R-113

- Which is false statement about multistage compression?

(A) Power consumption per unit of air delivered is low

(B) Volumetric efficiency is high

(C) It is best suited for compression ratios around 7 : 1

(D) The moisture in air is condensed in the intercooler

- Which of the following are the reasons for the volumetric efficiency of reciprocating compressor being less than 100%?

- Deviations from isentropic process.

- Pressure drop across the valves.

- Superheating in compressor.

- Clearance volume.

- Deviations from isothermal process.

- Leakages.

Select the correct answer from the codes given below :
Codes :
(A) 1, 2, 3 and 5
(B) 2, 3, 4 and 5
(C) 1, 4, 5 and 6
(D) 2, 3, 4 and 6.

- Pressure ratio in gas turbines is the ratio of

(A) compressor pressure ratio

(B) highest pressure to exhaust pressure

(C) inlet pressure to exhaust pressure

(D) pressures across the turbine

- In regenerative air preheater, the heat is transferred

(A) from a metal wall from one medium to another

(B) from heating an intermediate material and then heating the air from this material

(C) by direct mixing

(D) heat is transferred by bleeding some gas from furnace

- Overfire burning is the phenomenon of

(A) supply of excess air

(B) supply of excess coal

(C) burning CO and unburnts in upper zone of furnace by supplying more air

(D) fuel bed firing

- The ratio of limiting friction and normal reaction is known as

(A) coefficient of friction

(B) angle of friction

(C) angle of repose

(D) sliding friction

- A particle moves along a straight line such that distance (x) traversed in t seconds is given by x=t2(t−4), the acceleration of the particle will be given by the equation

(A) 3t2−2t

(B) 3t2+2t

(C) 6t−8

(D) 6t−4

- When P is the effort and W is the load then linear law of machines is given by the relation

(A) W=mP+c

(B) W=mP−c

(C) P=mW−c

(D) P=mW+c

where m and c are constants

- Klein's construction can be used when

(A) crank has a uniform angular velocity

(B) crank has non-uniform velocity

(C) crank has uniform angular acceleration

(D) crank has uniform angular velocity and angular acceleration

- Fluctuation of energy of an engine is the

(A) variation of energy above and below the mean resisting torque line

(B) ratio of maximum and minimum energies

(C) difference between the maximum and minimum energies

(D) ratio of the maximum fluctuation of energy to the work done per cycle

- The valve rod in a steam engine is connected to an eccentric rod by

(A) cotter joint

(B) bolted joint

(C) kunckle joint

(D) universal coupling

- A screw is specified by

(A) major diameter × pitch

(B) minimum diameter × length

(C) major diameter × length

(D) mean diameter × pitch

- A riveted joint may fail due to

(A) failure of the rivets

(B) failure of the plates

(C) either failure of the rivets or failure of the plates

(D) failure of both rivets or plates

- The purpose of sprue down is to

(A) feed the casting at a rate consistent with the rate of solidification

(B) act as a reservoir for molten metal

(C) help feed the casting until all solidification takes place

(D) feed molten metal from pouring basin to gate

- Injection moulding is the ideal method of processing

(A) plastics

(B) thermo-setting plastics

(C) thermoplastics

(D) non-ferrous materials