|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ECE |
| --- | --- |

B

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : ECE |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- In a PLL, the loop filter affects:

A. Capture range and stability

B . Only capture range

C.  Only lock range

D. None

- 2. The electromagnetic wave impedance in free space is approximately:

A.    50 Ω

B.  100 Ω

C.   377 Ω

D.  200 Ω

- 3   For a stable discrete-time system, all poles must lie:

A. Inside the unit circle

B.  On the unit circle

C. Outside the unit circle

D.  On real axis

4. The MOSFET operates in saturation when:

A. Vgs < Vth

B. Vds > (Vgs − Vth)

C. Vds > (Vgs − Vth)

D. Vgs =0

- 5. The phase margin of a system with gain crossover frequency 10 rad/s and phase −135° is:

A. 45°

B. 30°

C. 60°

D. 15°

- 6. In a 12-bit DAC, the resolution (as a percentage of full scale) is:

A. 0.024%

B.  0.012%

C. 0.048%

D.  0.25%

- 7. The probability of bit error for BPSK in AWGN channel is given by:

A.  Q(√(2Eb/N0))

B.  1 − Q(√(2Eb/N0))

C.  Q(Eb/N0)

Answer: A

- 8.  For a signal  , its ROC is:

A. |z| > 0.5

B. |z| < 0.5

C.  |z| = 0.5

D.  |z| = 1

- 9. The efficiency of a class C amplifier is typically:

A.  25%

B.  50%

C.  78.5%

D.  >90%

10. A DSBSC transmitter radiates 1 kW when the modulating percentage is 60%.How much of carrier power is required if we want to transmit the same message by an AM transmitter?

- 5.56 kW

- 4.56 kW

- 3.23 kW

- 7.12 kW

- 11.  In a 2-port network, reciprocity implies:

A.  Z12 = Z21

B.  Y12 = −Y21

C.  h12 = −h21

D.  ABCD matrix symmetric

- 12. The cut-off  wavelength for TE10 mode in a rectangular waveguide with a = 2 cm is:

A. 1 cm

B.  2 cm

C. 4 cm

D. 10 cm

- 13.  The entropy H(X) of a binary source with probabilities (0.8, 0.2) is:

A. 0.72 bits

B. 1 bit

C. 0.5 bits

D. 0.28 bits

- 14. The Doppler shift for a radar operating at 10 GHz detecting a target moving at 30 m/s is:

A. 1 kHz

B. 2 kHz

C. 3 kHz

D. 10 kHz

- 15.  For a transmission line with Z0=50 Ω, length λ/4, terminated in 200 Ω, the input impedance is:

A. 50 Ω

B. 12.5 Ω

C. 100 Ω

D. 25 Ω

16. The antenna current of an AM broadcast transmitter ,modulated to a depth of 40 percent by an audio sine wave is 11 A.It increases to 12 A as a result of simultaneous modulation by another audio sine wave. What is the modulation index due to this second wave.

- 0.643

- 0.757

- 0.123

- 0.346

17.  An DSB-AM transmitter has carrier signal frequency of  1000 KHz.If the modulating signal frequency is 8000 Hz, What is the bandwidth of modulated  signal?

- 16 kHz

- 10 kHz

- 18 kHz

- 12 kHz

18. In an FM system, the audio frequency is 500 Hz and frequency deviation is 4.8 kHz.What is the modulation index?

- 9.6

- 8.2

- 7.5

- 6.2

19. In an FM system, the audio frequency is 500 Hz and frequency deviation is 14.4 kHz.What is the modulation index?

- 28.8

- 20.02

- 27.9

- 25.62

20. If  Ec=15 Volts and Em=5 Volts , what would be the modulation index of AM signal?

A.   0.33

B.   0.6

C.   0.4

D.  0.65

21. In delta modulation, the granular noise can be reduced by

- decreasing the step size

- decreasing the granular noise

- decreasing the sampling noise

- increasing the step size

22. A band limited signal is sampled at the Nyquist rate. The signal can be recovered by passing the samples through

- An LC filter

- PLL

- RLC filter

- low-pass filter with the appropriate bandwidth

23. A message signal having maximum frequency component of 1 kHz is sampled.What would be sampling rate?

A . 2 kHz

B.    4 kHz

- 8 kHz.

- 12 kHz

- 24.  In a BJT differential amplifier, the common-mode rejection ratio (CMRR) increases if:

A.  Tail current increases

B. Tail resistance increases

C.  Load resistance decreases

D.  β decreases

- 25.  In OFDM, orthogonality between subcarriers is maintained by:

A. Frequency spacing equal to symbol rate

B. Frequency spacing equal to inverse of symbol duration

C. Using same carrier frequency

D. Equal amplitude