|  | # FACULTY OF _____________________________ # DEPARTMENT OF ___________________________ |
| --- | --- |

A

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Subject-Specific Questions

(25×1=25)

### 1. Which of the following is not an advantage of CMOS over TTL?

### A. Low static power consumption
B. High packing density
C. High noise margin
D. High power dissipation

### Answer: D

- 2. In a feedback amplifier, if open loop gain = 100 and feedback factor = 0.01, the closed-loop gain is:

- A.  10

- B.  50

- C.  99

- D. 100

Answer: A

- 3. In a BJT differential amplifier, the common-mode rejection ratio (CMRR) increases if:

- A. Tail current increases

- B. Tail resistance increases

- C. Load resistance decreases

- D. β decreases

Answer: B

- 4. The efficiency of a class C amplifier is typically:

- A. 25%

- B. 50%

- C. 78.5%

- D.  >90%

Answer: D

5.  In a silicon diode, the forward voltage drop at room temperature is approximately:

- A. 1 V

- B. 100 mV

- C. 0.7 V

- D. 10 V

Answer: C.

6. The voltage gain of an inverting amplifier is (–100). If input voltage is 20 mV, output voltage is:

- A. 1 V

- B. 100 mV

- C. 0.7 V

- D. -2 V

Answer: D

7. The Boolean expression AB + A'B simplifies to:

- A. AB + A'B

- B. B

- C. AB

- D. A

Answer: B

8. The number of flip-flops required to design a mod-16 counter is:

- A.  10

- B.  4

- C.  1

- D. 2

Answer: B

9. A 400 watt carrier is modulated to a depth of 75 percent. Calculate the total power in the AM modulated wave.

- 512.5W

- 250W

- 400.5W

- 250.5W

Answer: A

### 10. The spectral efficiency of 16-QAM is higher than BPSK because…..

### A. It uses lower bandwidth
B. It transmits more bits per symbol
C. It has lower BER
D. It uses phase only

### Answer: B

### 11. A system described by  is….

### A. Non-linear and time-invariant
B. Linear and causal
C. Non-causal and stable
D. Time-variant

### Answer: B

### 12. Which of the following is not an advantage of CMOS over TTL?

### A. Low static power consumption
B. High packing density
C. High noise margin
D. High power dissipation

### Answer: D

### 13. A unity feedback system has open-loop transfer function  ​. The value of K for critical damping is…

### A. 1
B. 4
C. 8
D. 16

### Answer: C

### 14. In a digital communication system, increasing the roll-off factor of a raised cosine filter will…

### A. Increase bandwidth
B. Improve timing jitter tolerance
C. Reduce ISI
D. Sharpen the frequency cut-off

### Answer: A

### 15. In an 8-bit successive approximation ADC with a clock of 1 MHz, the total conversion time is approximately…

### A) 8 μs
B) 1 μs
C) 0.125 μs
D) 64 μs

### Answer: A

### 16. For a Butterworth low-pass filter of order N, the magnitude response is maximally flat at:

### A) Stopband only
B) Cut-off frequency
C) Origin
D) All frequencies

### Answer: B

### 17. A signal occupies 10 kHz bandwidth. The minimum sampling rate to ensure perfect reconstruction is….

### A) 10 kHz
B) 50 kHz
C) 15 kHz
D) 20 kHz

### Answer: D

### 18. A signal  is sampled at 200 Hz. The resulting sampled signal will exhibit….

### A) Aliasing of both components
B) No aliasing
C) Aliasing of one component
D) Cannot be determined without Nyquist rate

### Answer: C

### 19. The phase margin of a system is measured at the frequency where…?

### A. Gain is 0 dB
B. Phase is -180°
C. Phase is 0°
D. Gain is maximum

### Answer: A

### 20. Which is a property of a linear time-invariant (LTI) system?

### A. Memoryless
B. Boundedness
C. Shift-invariance
D. Non-causality

### Answer: C

### 21.  Which is not an application of a Schmitt trigger?

### A. Noise immunity
B. Waveform shaping
C. Analog to digital conversion
D. RF amplification

### Answer: D

### 22. The system described by   is…

### A. Linear and time-invariant
B. Non-linear and time-invariant
C. Linear but time-variant
D. Non-linear and time-variant

### Answer: C

### 23. For a signal , minimum Nyquist rate is…

### A. 100 Hz
B. 200 Hz
C. 400 Hz
D. 800 Hz

### Answer: D

### 24.  For a second-order system, when the damping ratio ζ=1, the system is…

### A. Under damped
B. Critically damped
C. Over damped
D. Oscillatory

### Answer: B

### 25. The gain of an inverting op-amp is  10. The input resistance is 10 kΩ. The feedback resistance is…

### A. 100 kΩ
B. 100 Ω
C. 10 kΩ
D. 90 kΩ

### Answer: A

Answer Key

A

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 | D | 26 |  |
| 2 | A | 27 |  |
| 3 | B | 28 |  |
| 4 | D | 29 |  |
| 5 | C | 30 |  |
| 6 | D | 31 |  |
| 7 | B | 32 |  |
| 8 | B | 33 |  |
| 9 | A | 34 |  |
| 10 | B | 35 |  |
| 11 | B | 36 |  |
| 12 | D | 37 |  |
| 13 | C | 38 |  |
| 14 | A | 39 |  |
| 15 | A | 40 |  |
| 16 | B | 41 |  |
| 17 | D | 42 |  |
| 18 | C | 43 |  |
| 19 | A | 44 |  |
| 20 | C | 45 |  |
| 21 | D | 46 |  |
| 22 | C | 47 |  |
| 23 | D | 48 |  |
| 24 | B | 49 |  |
| 25 | A | 50 |  |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.