|  | # FACULTY OF # ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING |
| --- | --- |

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | B : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Research Methodology

(25×1=25)

Subject-Specific Questions

(25×1=25)

1. The bandwidth of an FM signal with a maximum frequency deviation of 75 kHz and maximum modulating frequency of 15 kHz is approximately:

A. 180 kHz

B. 225 kHz

C. 300 kHz

D. 450 kHz

2. Which of the following materials has the smallest energy band gap at room temperature?

A. Silicon (Si)

B. Germanium (Ge)

C. Gallium Arsenide (GaAs)

D. Silicon Carbide (SiC)

3. In a PCM system with n = 8 bits, the signal-to-quantization noise ratio (SQNR) is approximately:

A. 36 dB

B. 42 dB

C. 50 dB

D. 60 dB

4. In an intrinsic semiconductor at equilibrium, the concentration of electrons is:

A.  Zero

B. Equal to the concentration of holes

C. Much greater than the hole concentration

D. Much less than the hole concentration

5. A carrier c(t) = 10cos(2πfc t) is modulated by m(t) = 2cos(2πfm t). The modulation index is:

A. 0.1

B. 0.2

C. 0.5

D. 2.0

6. A binary FSK system uses two frequencies f₁ = 1200 Hz and f₂ = 2400 Hz with bit rate = 1200 bps. Bandwidth is:

A) 1200 Hz

B) 2400 Hz

C) 3600 Hz

D) 4800 Hz

7. Which of the following contributes to current flow in a semiconductor?

A. Drift current only

B. Diffusion current only

C. Both drift and diffusion currents

D. Neither drift nor diffusion currents

8. Solar cells work on the principle of:

A. Photoconductivity

B. Photovoltaic effect

C. Thermionic emission

D. Photoelectric effect

9. In node voltage analysis, the number of independent node voltages is:

A. Number of nodes

B. Number of nodes minus one

C. Number of meshes

D. Number of branches minus number of nodes

10. A resistor at temperature T = 300 K has bandwidth B = 1 MHz Thermal noise power (N = kTB) is: [k = 1.38 × 10⁻²³ J/K]

A. 4.14 × 10⁻¹⁵ W

B. 4.14 × 10⁻¹² W

C. 1.38 × 10⁻¹⁵ W

D. 1.38 × 10⁻¹² W

11. Which of the following phenomena helps to determine the concentration and mobility of charge carriers?

A. Hall Effect

B. Photoelectric Effect

C. Compton Effect

D. Doppler Effect

12. The characteristic impedance of a lossless transmission line depends on:

A. Resistance and inductance

B. Inductance and capacitance

C. Resistance and capacitance

D. Resistance only

13. If a message signal of peak amplitude 2 V modulates a carrier of peak amplitude 10 V in AM, what is the modulation index?

A. 0.2

B. 0.5

C. 1.0

D. 5.0

14. The condition that decides the oscillator’s output frequency is:

A. Loop gain should at least be unity

B. Loop phase shift should be zero or integral multiple of 2π radians

C. Loop gain should be precisely unity

D. Loop phase shift should be precisely zero radian

15. The Nyquist rate for a signal with bandwidth 5 kHz is:

A. 5 kHz

B. 10 kHz

C. 2.5 kHz

D. 20 kHz

16. An antenna behaves as a resonant circuit only when its length is:

A. λ/2

B. λ/4

C. λ/2 or its integral multiple

D. None of these

17. Simplify: F = AB + A'C + BC

A. AB + A'C + BC

B. AB + A'C

C. A'C + BC

D. AB + BC

18. A 4:1 MUX has selected lines S1, S0 and inputs I0, I1, I2, I3. For S1=1, S0=0, the output is:

A. I0

B. I1

C. I2

D. I3

19. A 4-bit Serial-In Serial-Out (SISO) shift register receives input 1011 (MSB first). After 4 clock pulses, the output is:

A. 0000

B. 1111

C. 1011

D. 1101

20. In a MOSFET, the channel is formed by:

A. Doping only

B. Applying gate voltage

C. Applying drain voltage

D. Applying source voltage

21. In a BJT transistor, the base region is:

A. Heavily doped and thick

B. Lightly doped and thin

C. Heavily doped and thin

D. Lightly doped and thick

22. In a communication system, the matched filter maximizes:

A. Signal power

B. Bandwidth

C. Signal-to-noise ratio (SNR)

D. Bit rate

23. According to the Nyquist sampling theorem, the minimum sampling frequency for a signal with maximum frequency component fm is:

A. fm

B. 4fm

C. m/2

D. 2fm

24. A D Flip-Flop stores the input D at the clock edge. If D=1 and Q=0 before the clock edge, after the clock edge:

A. Q=1, Q'=0

B. Q=0, Q'=1

C) Q=0, Q'=0

D) Q=1, Q'=1

25. The depletion region width in a P-N junction increases with:

A. Forward bias

B. Reverse bias

C. Temperature

D. Doping concentration

Answer Key

B

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 |  | 26 | D |
| 2 |  | 27 | B |
| 3 |  | 28 | C |
| 4 |  | 29 | B |
| 5 |  | 30 | B |
| 6 |  | 31 | C |
| 7 |  | 32 | C |
| 8 |  | 33 | B |
| 9 |  | 34 | B |
| 10 |  | 35 | A |
| 11 |  | 36 | A |
| 12 |  | 37 | B |
| 13 |  | 38 | A |
| 14 |  | 39 | B |
| 15 |  | 40 | B |
| 16 |  | 41 | C |
| 17 |  | 42 | B |
| 18 |  | 43 | C |
| 19 |  | 44 | C |
| 20 |  | 45 | B |
| 21 |  | 46 | B |
| 22 |  | 47 | C |
| 23 |  | 48 | D |
| 24 |  | 49 | A |
| 25 |  | 50 | B |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using Math Type or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.