SRM Institute of Science and Technology

Faculty of Engineering and Technology

Ramapuram, Chennai, Tamil Nadu—600089

Department of Biomedical Engineering

PhD Entrance Examination, November 2025

Set:A

Test: PhD Entrance Examination                                                                  Date: 05/11/2025

Duration: 2 Hours                                                                                          Max. Marks: 100

Name:                                                                      Application No.

Instructions: Answer all the questions | Write the correct option in last page

| Q.No | Questions |
| --- | --- |
| 1 | The pulse oximeter sensor typically measures oxygen saturation by utilizing which two types of light? A) Infrared and ultraviolet B) Red and infrared C) Blue and green D) Yellow and violet |
| 2 | Which type of electrode is commonly used in electrocardiography (ECG)? A) Silver chloride electrode B) Tungsten electrode C) Copper electrode D) Gold electrode |
| 3 | In a patient monitoring system, which parameter is NOT typically measured? A) Blood pressure B) Respiratory rate C) Lung capacity D) Heart rate |
| 4 | A device that measures lung function by assessing the amount of air exhaled and inhaled is called a: A) Spirometer B) Sphygmomanometer C) Thermocouple D) Electroencephalograph |
| 5 | A clinical thermometer that measures body temperature by infrared sensing at the forehead is known as a: A) Digital thermometer B) Oral thermometer C) Ear thermometer D) Infrared forehead thermometer |
| 6 | Which frequency range is typically tested in standard pure tone audiometry? A) 20 to 200 Hz B) 100 to 1,000 Hz C) 250 to 8,000 Hz D) 10,000 to 20,000 Hz |
| 7 | Biomechanical analysis of the lower extremities is crucial for understanding: A) Arm swing in running B) Postural stability and gait C) Hand dexterity D) Eye movement |
| 8 | Bone growth in response to stress is best explained by: A) Wolff’s Law B) Hooke’s Law C) Pascal’s Principle D) Bernoulli’s Principle |
| 9 | Which spinal region is most susceptible to injury due to its load-bearing role? A) Cervical B) Thoracic C) Lumbar D) Sacral |
| 10 | In biomechanics, the term “pronation” refers to: A) Rotation of the forearm so the palm faces down B) Rotation of the forearm so the palm faces up C) Movement of the arm away from the body D) Movement of the arm toward the body |
| 11 | The movement that involves bending sideways at the waist is called: A) Flexion B) Extension C) Lateral flexion D) Rotation |
| 12 | In linear kinetics, momentum is calculated as: A) Mass divided by velocity B) Mass times velocity C) Force times acceleration D) Weight times speed |
| 13 | An object will continue in a state of rest or uniform motion unless acted upon by an external force. This statement refers to: A) Newton’s First Law B) Newton’s Second Law C) Newton’s Third Law D) Law of Conservation of Energy |
| 14 | The point at which the mass of an object is evenly distributed is known as: A) Center of gravity B) Center of pressure C) Center of mass D) Center of rotation |
| 15 | The term “range of motion” refers to: A) The force exerted by muscles B) The full movement potential of a joint C) The angle of movement in bones D) The velocity of motion |
| 16 | The movement of turning the sole of the foot inward is called: A) Eversion B) Inversion C) Pronation D) Supination |
| 17 | Which component of a laser determines the wavelength of the emitted light? a) Optical cavity b) Gain medium c) Power source d) Beam splitter |
| 18 | In a laser, the optical cavity usually consists of two mirrors. What is the role of one of these mirrors being partially transparent? a) It increases the laser's coherence b) It allows the amplified light to exit as a laser beam c) It reduces the intensity of the laser d) It keeps the laser beam monochromatic |
| 19 | Which type of laser-tissue interaction is primarily responsible for laser surgeries involving cutting and ablation? a) Photothermal b) Photochemical c) Photomechanical d) Photodynamic |
| 20 | In fluorescence-based diagnostics, which role does the laser play? a) It excites specific molecules in tissues to emit light b) It heats the tissue to create a contrast c) It is absorbed by pigments to generate an image d) It measures the mechanical properties of tissue |
| 21 | Photodynamic therapy (PDT) involves three main components: a light source, a photosensitizer, and __________ a) Oxygen b) Water c) Nitrogen d) Carbon dioxide |
| 22 | _________interaction type is characterized by mechanical effects such as cavitation or shock waves within tissue? a) Photothermal interaction b) Photochemical interaction c) Photomechanical interaction d) Photodynamic interaction |
| 23 | ____________type of tissue response is expected when a laser’s energy is primarily absorbed by water within the tissue? a) Coagulation b) Vaporization c) Pigmentation d) Photochemical reaction |
| 24 | ________ therapeutic device is commonly used to treat patients with irregular heart rhythms a) Ventilator b) Defibrillator c) Pacemaker d) Ultrasound machine |
| 25 | ___________imaging technique uses strong magnetic fields and radio waves to produce detailed images of organs and tissues a) X-ray b) CT scan c) MRI d) Ultrasound |