|  | # FACULTY OF # ENGINEERING & TECHNOLOGY # DEPARTMENT OF MECHANICAL ENGINEERING |
| --- | --- |

B

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Research Methodology

(25×1=25)

1. In simple random sampling, every member of the population:
A) Has an equal chance of being selected
B) Has a fixed probability of being excluded
C) Is grouped into subgroups
D) Is divided into strata based on characteristics

2. One of the main concerns when evaluating the originality of a thesis is:
A) Whether the research topic has been studied before
B) Whether the researcher has clearly followed others' conclusions without modification
C) Whether the research presents new findings or contributes to the existing body of knowledge
D) Whether the research uses only well-known, widely cited theories

3. The chi-square distribution is primarily used for:
A) Modeling the distribution of sums of squared normal variables
B) Modeling the number of events in a fixed interval of time
C) Calculating the probability of success in a fixed number of trials
D) Modeling continuous data with a fixed mean and variance

4. Exponential smoothing differs from moving average forecasting because it:
A) Gives equal weight to all data points
B) Gives more weight to recent observations than older ones
C) Requires the calculation of seasonal indices
D) Uses time-dependent variables in forecasting

5. A synopsis of a research project is typically:
A) A detailed explanation of the research findings
B) A brief summary of the research problem, objectives, methodology, and expected outcomes
C) A personal opinion on the research topic
D) A list of all references used in the study

6. The primary aim of validation in FEA is to:
A) Reduce mesh size
B) Compare simulation with experiments
C) Improve citations
D) Increase runtime

7. Optimization through simulation is useful because it allows engineers to:
A) Identify the best possible design parameters through iterative testing without physical prototypes
B) Simulate only the worst possible scenarios
C) Design systems with the lowest computational cost
D) Use random variables to find the optimum solution

8. In regression analysis, the slope of the regression line represents:
A) The predicted value of the dependent variable
B) The strength of the correlation between the variables
C) The rate of change of the dependent variable for each unit change in the independent variable
D) The starting value of the dependent variable

9. In multibody dynamics (MBD) simulations, the focus is on:
A) Simulating the fluid flow within a pipe system
B) Simulating the behavior of solid bodies under thermal stresses
C) Analyzing the motion and interaction of multiple interconnected rigid or flexible bodies
D) Calculating the impact resistance of materials

10. A feasible solution in optimization is one that:
A) Satisfies all the constraints of the problem
B) Maximizes the objective function
C) Minimizes the objective function
D) Has no constraints

11. The t-test is primarily used to:
A) Compare means between two or more independent groups
B) Test the association between two categorical variables
C) Compare the variances between two groups
D) Test the difference in means between two groups

12. In random sampling, the selection of each individual is:
A) Based on the researcher’s judgment
B) Based on the population’s characteristics
C) Completely random, with every individual having an equal chance of being selected
D) Based on a specific subgroup

13. Modeling in mechanical engineering involves:
A) The physical construction of a system to analyze
B) Representing a real-world system through mathematical equations
C) Observing the behavior of the system without using any equations
D) None of the above

14. The best research problem is one that is:
A) Impossible to measure
B) Relevant, novel, and feasible
C) Extremely broad
D) Based only on assumptions

15. In regression analysis, the dependent variable is also known as:
A) The predictor variable
B) The independent variable
C) The response variable
D) The outcome of correlation

16. Which of the following distributions is commonly used to model the number of successes in a fixed number of trials, where each trial has the same probability of success?
A) Poisson distribution
B) Exponential distribution
C) Binomial distribution
D) Normal distribution

17. A oral presentation at a conference typically:
A) Involves the display of research findings on a large poster board
B) Includes a verbal presentation followed by a Q&A session with the audience
C) Is limited to visual aids and does not involve speaking
D) Is a written document that is handed to attendees

18. In reliability engineering, failure probability is associated with:
A) Mean only
B) Risk and uncertainty
C) Mesh quality
D) Copyright law

19. The purpose of a mathematical model in mechanical engineering is to:
A) Analyze the behavior of a physical system without building it
B) Replace the real system with a physical model
C) Provide an exact replica of the physical system
D) Only to identify the materials used

20. Optimization is primarily concerned with:
A) Finding the worst solution to a problem
B) Identifying the best solution given constraints
C) Finding the average solution to a problem
D) Determining the range of possible solutions

21. Which of the following forecasting methods requires no historical data?
A) Delphi method
B) Time series analysis
C) Moving average
D) Exponential smoothing

22. Which of the following is NOT a common criterion for evaluating a thesis?
A) The clarity of the research problem
B) The quality and thoroughness of the literature review
C) The researcher’s ability to summarize other researchers’ opinions
D) The originality and significance of the research findings

23. The purpose of validation in simulation models is to:
A) Test the accuracy of the model by comparing it with real-world results
B) Create new models based on assumptions
C) Increase the computational speed of the model
D) Simplify the simulation equations

24. Simulation in mechanical engineering refers to:
A) The physical testing of materials
B) The process of solving real-world problems through computational models
C) Constructing the actual mechanical system
D) Generating experimental data manually

25. In evaluating the citations and references in a thesis, examiners typically check:
A) Whether all sources used in the thesis are correctly cited and referenced according to the required style guide
B) Whether the references are directly copied from online sources
C) The number of sources referenced, regardless of relevance
D) Whether the sources include a variety of popular websites

Subject-Specific Questions

(25×1=25)

26. Hobbing is mainly used for:
A) shaft
B) gear cutting
C) drilling
D) grinding

27. The primary function of a jig is to:
A) hold work
B) guide tool
C) measure part
D) polish surface

28. Reaming is used for:
A) rough hole
B) sizing hole accurately
C) cutting threads
D) knurling

29. The M-code M03 indicates:
A) coolant off
B) spindle ON clockwise
C) stop
D) tool change

30. STL file is used in:
A) CNC code
B) additive manufacturing
C) heat balance
D) FEA mesh only

31. The temperature point at which the change starts on heating the steel is called:
A) lower critical point
B) upper critical point
C) point of recalescence
D) point of decalescence

32. Structural sections such as rails, angles, I-beams are made by:
A) hot rolling
B) hot drawing
C) hot piercing
D) hot extrusion

33. Grinding wheel grade indicates:
A) grain size
B) bond strength
C) speed
D) finish

34. In electro-discharge machining, the tool is made of:
A) Brass
B) Copper
C) copper tungsten alloy
D) all of these

35. A set of eight form relieved milling cutters for each module is provided to enable cutting of gears of different:
A) materials
B) types of gears
C) number of teeth
D) width of gears

36. The operation of cutting of a flat sheet to the desired shape is called:
A) Shearing
B) Piercing
C) Punching
D) Blanking

37. Which of the following statements is correct as regard to up milling?
A) It can not be used on old machines due to backlash between the feed screw of the table and the nut.
B) The chips are disposed off easily and do not interfere with the cutting.
C) The surface milled appears to be slightly wavy.
D) The coolant can be poured directly at the cutting zone where the cutting force is maximum.

38. The chip thickness ratio is used in:
A) Merchant analysis
B) FEA
C) CFD
D) casting

39. Design against creep is important for:
A) high temperature parts
B) shafts
C) keys
D) springs

40. Beam deflection is maximum at fixed end in:
A) cantilever
B) simply supported
C) overhanging
D) fixed-fixed

41. Contact stress is critical in:
A) gears
B) boilers
C) springs
D) nozzles

42. Fracture toughness is used in:
A) crack design
B) thermal design
C) lubrication
D) casting

43. Superheating in vapor compression cycle generally:
A) increases refrigerating effect
B) decreases COP always
C) no effect
D) reduces pressure

44. Bilateral tolerance means variation allowed on:
A) one side only
B) both sides of nominal size
C) actual size only
D) shaft only

45. Draft allowance is provided in casting for:
A) shrinkage
B) easy pattern withdrawal
C) machining
D) wear

46. The lathe spindles are usually made hollow and provided with:
A) internal taper
B) external taper
C) internal and external taper
D) no taper

47. The point of recalescence in steel heating refers to:
A) lower critical point
B) upper critical point
C) point of recalescence
D) point of decalescence

48. Hot extrusion is used for manufacturing:
A) rails and angles
B) I-beams
C) shafts
D) structural sections

49. Punching and blanking are operations used in:
A) sheet metal cutting
B) turning
C) milling
D) grinding

50. Machining allowance in casting is provided for:
A) shrinkage
B) pattern withdrawal
C) machining
D) wear

Answer Key

B

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 | B | 26 | B |
| 2 | B | 27 | B |
| 3 | B | 28 | B |
| 4 | B | 29 | B |
| 5 | B | 30 | B |
| 6 | A | 31 | A |
| 7 | A | 32 | A |
| 8 | B | 33 | B |
| 9 | D | 34 | D |
| 10 | C | 35 | C |
| 11 | A | 36 | A |
| 12 | C | 37 | C |
| 13 | A | 38 | A |
| 14 | A | 39 | A |
| 15 | A | 40 | A |
| 16 | A | 41 | A |
| 17 | A | 42 | A |
| 18 | A | 43 | A |
| 19 | B | 44 | B |
| 20 | B | 45 | B |
| 21 | A | 46 | A |
| 22 | C | 47 | C |
| 23 | A | 48 | D |
| 24 | B | 49 | A |
| 25 | C | 50 | C |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.