|  | # FACULTY OF ALLIED HEALTH SCIENCES # MEDICAL IMAGING TECHNOLOGY |
| --- | --- |

A

A

| Name | :___________________________ | Department | : Medical Imaging Technology |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to Candidates - This question paper consists of Multiple-Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Subject-Specific Questions (25×1=25)

1. Which sequence is most effective in minimizing the effects of magnetic field inhomogeneity?

- Gradient echo

- Spin echo

- Echo planar imaging

- Diffusion-weighted imaging

- According to the International Commission on Radiological Protection (ICRP), which principle ensures that the selection of the best protection option maximizes the margin of good over harm?

- Principle of justification

- Principle of limitation

- Principle of optimization (ALARA)

- Principle of personnel monitoring

- During "Parallel Imaging" (e.g., SENSE or GRAPPA), the "g-factor" (geometry factor) is a critical parameter. What does a higher g-factor specifically indicate?

- Improved spatial resolution

- Faster gradient switching speeds

- Increased regional noise amplification due to coil geometry

- Reduction in chemical shift artifacts

- Following the emission of a positron, the resulting annihilation radiation consists of two photons that are emitted simultaneously at approximately:

- 511 keV each, 180 degrees apart

- 1.02 MeV each, 90 degrees apart

- 140 keV each, 180 degrees apart

- 511 keV each, 45 degrees apart

- In Diffusion Tensor Imaging (DTI), the "Fractional Anisotropy" (FA) value of 0 would most likely be recorded in which environment?

- Corticospinal tract

- Corpus callosum

- Ventricular Cerebrospinal fluid (CSF)

- Optic nerve

- For biological tissues at clinical magnetic field strengths, what is the generally accepted relationship between the relaxation time constants?

- T2* > T2 > T1

- T1 > T2 > T2*

- T1 = T2 = T2*

- T2 > T1 > T2*

- The "Overhauser Effect" is utilized in certain advanced molecular imaging techniques to:

- Increase the signal-to-noise ratio via dynamic nuclear polarization

- Decrease the scan time of 3D-TOF MRA

- Eliminate flow-related enhancement

- Correct for respiratory motion in cardiac MRI

- Which frequency-dependent metric is the accepted standard for quantifying the overall dose efficiency and signal-to-noise ratio (SNR) performance of a digital imaging system?

- Modulation Transfer Function (MTF)

- Contrast-to-Noise Ratio (CNR)

- Detective Quantum Efficiency (DQE)

- Nyquist Frequency

- Which RF pulse sequence is designed to maintain a "Steady State" of both longitudinal and transverse magnetization?

- Fast Spin Echo (FSE)

- Gradient Recalled Echo (GRE)

- Coherent Balanced Steady-State Free Precession (bSSFP)

- Inversion Recovery (IR)

- In a quantum-limited Computed Tomography (CT) system, if the slice thickness is reduced from 5 mm to 0.625 mm while keeping all other technique factors constant, the image noise will:

- Decrease by a factor of 8

- Remain the same

- Increase by a factor of approximately 2.8

- Increase by a factor of 8

- Iterative Reconstruction" (IR) algorithms in CT are superior to "Filtered Back Projection" (FBP) primarily because they:

- Use less computational power

- Model the statistical noise and system optics to reduce dose while maintaining image quality

- Eliminate the need for a bow-tie filter

- Only require a 180-degree tube rotation

- Which parameter is defined as the average amount of energy deposited locally in an absorber per unit path length of a charged particle?

- Specific ionization

- Linear Energy Transfer (LET)

- Air Kerma

- Absorbed Dose

- In Dual-Source CT, what is the primary "Temporal Resolution" advantage when using two tubes mounted at 90-degree offsets?

- It doubles the scan length per second

- It allows for heart rate-independent imaging by requiring only a quarter-rotation for data acquisition

- It eliminates the need for iodine contrast

- It reduces the "Heel Effect"

- In Magnetic Resonance Imaging, the upper limit of detectable spatial frequency is determined by the sampling pitch (D) and is known as the:

- Larmor frequency

- Resonant frequency

- Nyquist frequency

- Precessional frequency

- The "Z-axis sampling" or "Overbeaming" in MDCT leads to an increase in:

- Spatial resolution

- Penumbra and total patient dose

- Hounsfield unit accuracy

- Cone-beam artifact correction

- In Positron Emission Tomography (PET), the virtual straight line connecting two detectors that simultaneously record a photon pair is termed the:

- Ray path

- Projection axis

- Line of response

- Isocentre

- When calculating the "Pitch" in a 128-slice CT scanner, the formula accounts for table feed per rotation divided by:

- The width of a single detector element

- The total collimation width

- The gantry diameter

- The scan length

- What physical phenomenon occurs when lower energy photons in a polyenergetic X-ray beam are preferentially attenuated by dense structures, causing an upward shift in the average beam energy?

- The Heel Effect

- Compton Scatter

- Beam Hardening

- Photoelectric Effect

- In Digital Subtraction Angiography (DSA), "Misregistration Artifacts" are mathematically corrected using which process?

- Logarithmic amplification

- Pixel shifting/Remasking

- Edge enhancement filtering

- Temporal averaging

- Which MR contrast mechanism utilizes off-resonance radiofrequency (RF) pulses to saturate protons bound to macromolecules, thereby suppressing background anatomic signal to enhance vascular visualization?

- Time-of-Flight (TOF)

- Phase contrast

- Diffusion-Weighted Imaging (DWI)

- Magnetization Transfer Contrast (MTC)

- The "Bernoulli Equation" used in Echocardiography/Doppler to calculate pressure gradients is a simplified version of the energy conservation law. Which factor is ignored in this simplification?

- Flow velocity

- Flow acceleration and viscous friction

- Vessel diameter

- Red blood cell mass

- In the structural shielding design of a diagnostic facility, the fraction of time that an area adjacent to the X-ray room is expected to be inhabited is the:

- Workload factor (W)

- Use factor (U)

- Occupancy factor (T)

- Attenuation factor (A)

- In Contrast-Enhanced Ultrasound (CEUS), "Non-linear oscillation" of microbubbles at a low Mechanical Index (MI) allows for:

  - Destruction of the bubbles for therapeutic delivery

  - Separation of the tissue signal from the microbubble signal (Pulse Inversion)

  - Increased thermal heating of the target lesion

  - High-speed cavitation

- The probability of photoelectric absorption per unit mass is approximately proportional to the cube of the atomic number (Z3) and inversely proportional to the:

- Square of the Energy (E2)

- Cube of the Energy (E3)

- Square root of the Energy

- Linear energy transfer

- Which ultrasound artifact is caused by the "Reflection" of the sound beam off a highly reflective curved interface, creating a false structure deeper than the real one?

- Shadowing

- Mirror image artifact

- Enhancement

- Anisotropy

Answer Key

A

A

| Question No. | Correct Option |
| --- | --- |
| 1 | B |
| 2 | A |
| 3 | C |
| 4 | A |
| 5 | C |
| 6 | B |
| 7 | A |
| 8 | C |
| 9 | C |
| 10 | C |
| 11 | B |
| 12 | B |
| 13 | B |
| 14 | C |
| 15 | B |
| 16 | C |
| 17 | B |
| 18 | C |
| 19 | B |
| 20 | D |
| 21 | B |
| 22 | C |
| 23 | B |
| 24 | B |
| 25 | B |