|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ECE |
| --- | --- |

A

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : ECE |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

Section B                                                                            25 Marks

1. An NPN transistor is operating in the active region. The base-emitter junction is forward biased with 0.7 V, and the collector-base junction is reverse biased. If the emitter current is 10 mA and current gain β=100, what is the collector current?

A) 0.1 mA

B) 9.9 mA

C) 10 mA

D) 1 A

2. Simplify the Boolean expression:
(A+B)(A+B′) 
A) A
B) B
C) 1
D) 0

3. Which of the following best describes BIBO stability for a continuous-time LTI system?

a) System is stable if all poles are in the left half of the complex plane

b) System is stable if any pole is on the jw axis

c) System is stable if there are no poles in the right half of the complex plane

d) Stability is independent of pole locations

4. The Fourier Transform of a rectangular pulse is:

a) Gaussian

b) Sinc function

c) Delta function

d) Ramp function

5. A traffic signal has GREEN for 70 seconds, YELLOW for 5 seconds, and RED for 75 seconds. The clock period is 5 seconds. What is the minimum number of flip-flops required to design the FSM?

A) 3

B) 4

C) 5

D) 6

6. What is the probability of getting at least one head in two tosses of a fair coin?

A) 1/4

B) 1/2

C) 3/4

D) 1

7. A bulb controlled by two switches on different floors, where either switch can turn the bulb ON or OFF independently, corresponds to which logic gate?

A) AND

B) OR

C) XOR

D) NAND

8. Thevenin’s theorem states that any linear two-terminal network can be replaced by:

A) A voltage source in series with a resistor

B) A current source in parallel with a resistor

C) A voltage source in parallel with a resistor

D) A current source in series with a resistor

9. Bayes theorem is used to calculate:

A) Joint probability

B) Marginal probability

C) Conditional probability

D) Independent probability

10. Probability of getting at least one head in two tosses of a fair coin is:

A) 1/4

B) 1/2

C) 3/4

D) 1

11. Which of the following is a voltage-controlled device?

A) SCR

B) FET

C) BJT

D) Diode

12. The cut-in voltage of a Germanium diode is:

A) 0.7 V

B) 0.3 V

C) 0.5 V

D) 1.0 V

13. If a silicon diode is forward biased in a circuit with 12 V supply and 240 Ω resistor, what is the voltage drop across the diode?

A) 0.3 V

B) 0.7 V

C) 1.2 V

D) 5 V

14. Which of the following is a trivalent doping element?

A) Phosphorus

B) Boron

C) Antimony

D) Arsenic

15. For normal active operation of an NPN transistor, the emitter-base junction must be:

A) Reverse biased

B) Forward biased

C) Either forward or reverse biased

D) No bias required

16. A message signal of peak value 2 V is used to modulate a carrier wave of peak value 10 V. The modulation index of the amplitude modulated wave is:

A) 0.2

B) 5

C) 0.5

D) 0

17. The bit rate of a digital communication system is R kbit/s and the modulation used is 32-QAM. The minimum bandwidth required for ISI-free transmission is:

A) R/10 Hz

B) R/10 kHz

C) R/5 Hz

D) R/5 kHz

18. A bit stream is partitioned into even and odd streams in which modulation scheme?

A) FSK

B) QPSK

C) MSK

D) BPSK

19. The variation of frequency of a carrier wave with respect to the amplitude of the modulating signal is called:

A) Amplitude modulation

B) Frequency modulation

C) Phase modulation

D) Pulse width modulation

20. The convolution of two signals in time domain corresponds to which operation in frequency domain?

A) Addition

B) Multiplication

C) Differentiation

D) Integration

21. A system is said to be causal if its output depends on:

A) Future inputs only

B) Past and present inputs only

C) Present inputs only

D) Past, present, and future inputs

22. S-parameters in transmission lines are used to:

A) Represent voltage and current

B) Describe power reflection and transmission at ports

C) Calculate resistance only

D) Measure capacitance

23. The Poynting vector represents:

A) Magnetic field intensity

B) Electric field intensity

C) Power flow per unit area

D) Charge density

24. When a plane wave is incident normally on a boundary between two media, the reflection coefficient depends on:

A) The frequency only

B) The intrinsic impedances of the two media

C) The angle of incidence

D) The polarization only

25. In communication systems, which of the following modes involves communication between a single transmitter and a single receiver?

(a) Broadcast communication

(b) Simplex communication

(c) Multipoint communication

(d) Point-to-point communication