|  | # FACULTY OF _____________________________ # DEPARTMENT OF __ELECTRICAL AND ELECTRONICS ENGINEERING__ |
| --- | --- |

B

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (name, application number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Subject-Specific Questions

(25×1=25)

      - A unit step current is applied to a parallel RLC circuit at t=0. The voltage across the circuit at t = 0+ is

- 1

- 1/R

- 0

- ∞

- For a balanced 3-phase system, the line current is 20 A and the line voltage is 400 V. If the power factor is 0.8 lagging, the total reactive power is

- 11.09 kVAR

- 13.86 kVAR

- 6.4 kVAR

- 8.31 kVAR

- In a series RLC circuit, at half-power frequencies, the current is ___ times the current at resonance

- 1.414

- 2

- 0.5

- 0.707

- A capacitor is charged to a voltage V1. At t = 0, it is discharged through a resistor R. If the energy stored in the capacitor is W1, what is the total energy dissipated in the resistor after infinite time?

  - W1

  - W1/2

  - 0

  - Cannot be determined

- A DC source has an internal resistance of 10 Ω. If a variable load resistor 𝑅 is connected, what is the efficiency of power transfer when 𝑅 is adjusted for maximum power?

  - 25%

  - 50%

  - 75%

  - 100%

- In a DC machine, if the brush axis is shifted from the Magnetic Neutral Axis in the direction of rotation, the effect on armature reaction is

  - Cross-magnetizing and demagnetizing

  - Cross-magnetizing and magnetizing

  - Purely magnetizing

  - Purely demagnetizing

- Which of the following is NOT a consequence of operating a transformer at a voltage higher than rated?

  - Increased core losses

  - Increased eddy current losses

  - Increased inrush current

  - Increased copper losses

- In a 3-phase, 4-pole, 50 Hz induction motor, the frequency of the rotor current is 2 Hz. The speed of the rotor in rpm is

  - 1440 rpm

  - 1480 rpm

  - 1460 rpm

  - 1420 rpm

- A 3-phase synchronous motor, while running, has one of its supply phases disconnected. The motor will

  - Stop immediately

  - Reverse its direction

  - Continue to run with reduced power and increased vibration

  - Run at double speed with increased vibration

- In a magnetic circuit, the magnetic field energy is mainly stored in

  - The air gap

  - The iron core

  - The windings

  - The yoke

- When an SCR is subjected to a high dv/dt, it may turn ON due to

  - Gate current flow

  - Anode current exceeding latching current

  - Capacitive current flowing through the junction

  - Thermally generated carriers

- A single-phase semi-converter is operating at a firing angle 𝛼=60∘ with a resistive load. The conduction angle for each SCR is

  - 30°

  - 60°

  - 90°

  - 120°

- A DC-DC converter has an efficiency of 90% and is stepping down 100V to 10V. If the output power is 100W, what is the input current?

  - 1 A

  - 1.1 A

  - 0.9 A

  - 10 A

- In a Current Source Inverter (CSI), what is the primary reason for using series-connected diodes with thyristors?

  - To reduce high switching transients

  - To increase inverter efficiency

  - To provide a path for freewheeling current

  - To reverse-bias the SCRs for commutation

- A half-controlled AC voltage controller feeding a resistive load leads to

  - Both odd and even harmonics in the output

  - Only odd harmonics in the output

  - Only even harmonics in the output

  - No harmonic generation

- A 3-phase, 3-wire system has 10 kV line voltage. The conductor diameter is 2 cm, and the spacing is 1 m. If the visual critical voltage is considered, what is the effect of using a larger diameter conductor?

  - Decreases the visual critical voltage

  - Increases the visual critical voltage

  - Does not affect the visual critical voltage

  - Reduces the corona loss immediately

- In a 2-wire DC distributor, if the voltage drop at the far end is 10 V with a concentrated load, what is the voltage drop if the same load is uniformly distributed over the same length?

  - 5 V

  - 10 V

  - 15 V

  - 20 V

- In a Hall effect sensor, to achieve a high output voltage for a given magnetic field, the semiconductor material should have

  - High carrier density and low mobility

  - Low carrier density and high mobility

  - High carrier density and high mobility

  - Low carrier density and low mobility

- In a chopper-fed DC motor, the load torque is constant. If the duty cycle 𝐷 is increased, the motor speed will

  - Becomes zero

  - Remains constant

  - Decreases

  - Increases

- SF₆ gas is used in circuit breakers because it is

  - Inert, low density

  - Electropositive, high cooling

  - Electronegative, high dielectric strength

  - Non-toxic, low ionization potential

- A second-order system has a damping ratio 𝜁= 0.5 and natural frequency 𝜔𝑛=4 rad/s. The settling time for a 2% tolerance band is

  - 1s

  - 2s

  - 4s

  - 8s

- The phase crossover frequency is the frequency at which the Nyquist plot crosses the

  - Real axis

  - Imaginary axis

  - Unit circle

  - 45° line

- Which of the following statements about PI controllers is true?

  - PI controllers reduce steady-state error

  - PI controllers improve transient response, making it faster

  - PI controllers are highly effective for rapid process load changes

  - PI controllers always eliminate overshoot

- In symmetrical component analysis, for a line-to-line fault at a bus, the zero-sequence network is

  - Connected in series with positive and negative sequence networks

  - Connected in parallel with positive and negative sequence networks

  - Not involved in the fault

  - Shorted to ground

- A PMMC instrument is used to measure a current 𝑖(𝑡)=5+3sin(𝜔𝑡)+2sin(2𝜔𝑡). The instrument has a full-scale deflection of 10 A. What is the deflection of the instrument?

  - 0 A

  - 5 A

  - 7 A

  - 10 A

Answer Key

B

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 | (Example:) A | 26 | C |
| 2 |  | 27 | D |
| 3 |  | 28 | D |
| 4 |  | 29 | A |
| 5 |  | 30 | B |
| 6 |  | 31 | A |
| 7 |  | 32 | D |
| 8 |  | 33 | A |
| 9 |  | 34 | C |
| 10 |  | 35 | A |
| 11 |  | 36 | C |
| 12 |  | 37 | D |
| 13 |  | 38 | B |
| 14 |  | 39 | D |
| 15 |  | 40 | A |
| 16 |  | 41 | B |
| 17 |  | 42 | A |
| 18 |  | 43 | B |
| 19 |  | 44 | D |
| 20 |  | 45 | C |
| 21 |  | 46 | B |
| 22 |  | 47 | A |
| 23 |  | 48 | A |
| 24 |  | 49 | C |
| 25 |  | 50 | B |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.