|  | # FACULTY OF ALLIED HEALTH SCIENCES # MEDICAL IMAGING TECHNOLOGY |
| --- | --- |

B

B

| Name | :___________________________ | Department | : Medical Imaging technology |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 1 hr |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (name, application number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Subject-Specific Questions  (25×1=25)

1. According to MRI safety guidelines, the administrative control limit for static magnetic fringe fields beyond which access must be strictly controlled is:

- 0.1mT

- 0.5mT

- 5.0mT

- 1.5mT

2. "Elastography" measures tissue stiffness by calculating the "Young's Modulus." In Shear Wave Elastography (SWE), stiffness is directly related to:

- The frequency of the probe

- The velocity of the induced transverse shear waves

- The depth of the focal zone

- The Pulse Repetition Period

3. For the energy region typically used in soft tissue CT imaging (40–60 keV effective), the Hounsfield Unit is primarily a map of which physical parameter?

- Proton density

- Mass density

- Electron density

- Effective atomic number

4. The "Wall Filter" in Color Doppler imaging is used to eliminate:

- High-velocity blood flow signals

- Low-frequency, high-amplitude signals from moving vessel walls

- Thermal noise from the transducer

- Aliasing artifacts

5. In which type of radioactive decay does a metastable nucleus transition to a lower energy state by emitting a gamma ray without changing its atomic or mass number?

- Beta-minus decay

- Electron capture

- Isomeric transition

- Positron emission

6. In PET-CT, "Time-of-Flight" (TOF) technology improves image quality by:

- Measuring the exact time difference between the detection of two coincidence photons to better localize the annihilation event

- Using faster X-ray tube rotation

- Reducing the half-life of Fluorine-18

- Eliminating the need for attenuation correction

7. Which Computed Tomography reconstruction paradigm allows for lower radiation doses by explicitly modelling the physical optics and geometry of the scanner?

  - Filtered Back projection (FBP)

  - Iterative Reconstruction (IR)

  - Simple Back projection

  - Fourier Transform reconstruction

8. Standardized Uptake Value (SUV) is best defined as:
          A. Activity per unit time

B. Tissue activity normalized to injected dose and body parameter

C. Photon count per second
          D. Metabolic rate of glucose

9. What term defines the probability that a vacancy in an inner atomic shell will result in the emission of a characteristic X-ray rather than an Auger electron?

  - Quantum yield

  - Photoelectric fraction

  - Stopping power

  - Fluorescent yield

10 . Which tissue has the highest tissue weighting factor?

- Skin

- Thyroid

- Bone marrow

- Brain

- In Nuclear Medicine, the "Gamma Camera" uses a "Collimator." If the hole length is increased, what is the effect on the system?

- Sensitivity increases, Resolution decreases

- Sensitivity decreases, Resolution increases

- Both Sensitivity and Resolution increase

- No change to either

- In Multi-Detector CT (MDCT), the practice of extending the X-ray beam slightly beyond the active detector arrays to avoid using the beam penumbra is called:

  - Over beaming

  - Adaptive collimation

  - Geometric efficiency

  - Z axis tracking

- In the context of Thermoluminescence Dosimetry (TLD) used for personnel monitoring, the process of "Fading" is mathematically characterized by the unintended release of trapped electrons. This phenomenon is primarily a result of:

- High energy photon interaction during readout

- Thermal stimulation at ambient temperatures over time

- The “Supralinearity” effect of the phosphour crystal

- Exposure to low energy non- ionizing RF interferance

- The "Oxygen Enhancement Ratio" (OER) in radiobiology is highest for which type of radiation?

- Alpha particles (High LET)

- Neutrons

- X Rays and gamma rays (Low LET)

- Protons

- Which electronic component in a nuclear medicine detection system is used to reject scattered photons by analysing the electrical signals for a specific energy window?

- Photomultiplier Tube (PMT)

- Preamplifier

- Pulse Height Analyzer (PHA)

- Analog-to-Digital Converter (ADC)

- In MRI of the brain, "Wallerian Degeneration" in the corticospinal tract following a chronic stroke is best visualized on T2-weighted images as:

- Increased signal intensity along the downstream tract

- Reduced diffusion in the acute phase

- Calcification of the internal capsule

- Enhancing nodules

- Which radiation protection quantity is calculated as the sum of weighted equivalent doses to specific tissues to account for their varying biological sensitivities?

- Absorbed dose

- Kerma

- Effective dose

- Exposure

- Which sign on a Chest CT is highly specific for "Invasive Aspergillosis" in neutropenic patients?

- Tree in bud sign

- CT halo sign

- Finger in glove sign

- Reverse Halo Sign

- Dual-Energy X-ray Absorptiometry (DEXA) systems can distinguish bone mineral density from surrounding soft tissue by exploiting the:

- Differences in T1 relaxation times

- Different energy dependencies of attenuation at two X-ray levels

- Reflection of high-frequency sound waves

- Speed of light in different mediums

- In MRI spectroscopy (MRS), a significant increase in the "Choline peak" relative to the "NAA peak" is a biomarker for:

- Neuronal loss/ Infarction

- High cellular turnover/ Malignacy

- Hepatic encephalopathy

- Normal aging

- Federal regulations in the United States specify that the maximum permissible leakage radiation at 1 metre from the focal spot of a diagnostic X-ray tube must not exceed:

- 10 mGy/h

- 1.0 mGy/h

- 0.88 mGy/h (100 mR/h)

- 50 mSv/year

- The "Relative Biological Effectiveness" (RBE) of radiation increases as the "Linear Energy Transfer" (LET) increases, up to a maximum of approximately:

- 10 keV/µm

- 1000 keV/µm

- 500 keV/µm

- 100 keV/µm

- The overall efficiency of a scintillator in capturing incident X-ray or gamma-ray photons based on its thickness and attenuation coefficient is defined as:

- Geometric Efficiency

- Conversion Efficiency

- Quantum Detection Efficiency (QDE)

- Detective Quantum Efficiency (DQE)

- What specific structural component is used to line the walls of an MRI suite to prevent external FM radiofrequency signals from interfering with image acquisition?

- Lead lining

- Borated polyethylene

- Faraday cage

- Concrete barrier

- In Digital Mammography, why are K-edge filters such as Molybdenum or Rhodium used?

- To selectively transmit characteristic X-rays for improved contrast

- To increase the heat capacity of the anode

- To eliminate the need for an ant scatter grid

- To increase the magnification factor

Answer Key

B

B

| Question No. | Correct Option |
| --- | --- |
| 1 | B |
| 2 | B |
| 3 | C |
| 4 | B |
| 5 | C |
| 6 | A |
| 7 | B |
| 8 | B |
| 9 | D |
| 10 | C |
| 11 | B |
| 12 | A |
| 13 | B |
| 14 | C |
| 15 | C |
| 16 | A |
| 17 | C |
| 18 | B |
| 19 | B |
| 20 | B |
| 21 | C |
| 22 | D |
| 23 | C |
| 24 | C |
| 25 | A |