|  | # College of Allied Health Sciences, SRMIST, TRC # Subject: Medical Imaging Technology |
| --- | --- |

B

Ph.D. Entrance Examination – November 2025

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MIT |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- The Larmor frequency is directly proportional to:
A) Gyromagnetic ratio only
B) Magnetic field strength
C) Echo time
D) Flip angle

- The most common digital detector used in DR systems is:
A) Image intensifier
B) CCD camera
C) Flat panel detector
D) Film-screen cassette

- The radionuclide commonly used in PET imaging is:
A) Tc-99m
B) F-18
C) I-131
D) Ga-67

- PET-CT combines:
A) Functional and structural imaging
B) Ultrasound and MRI
C) Bone scan and CT
D) X-ray and fluoroscopy

- The best material for shielding diagnostic X-rays is:
A) Copper
B) Aluminum
C) Lead
D) Tungsten

- The function of PACS in hospitals is to:
A) Store and retrieve images
B) Perform diagnosis
C) Control exposure
D) Generate X-rays

- DICOM stands for:
A) Digital Communication in Medicine
B) Digital Imaging and Communication in Medicine
C) Diagnostic Imaging Computer Method
D) Digital Image Computation Method

- Quantum mottle in digital imaging is caused by:
A) High exposure
B) Low exposure and insufficient photons
C) Overdevelopment
D) Excess filtration

- The annual effective dose limit for occupational radiation workers is:
A) 5 mSv
B) 10 mSv
C) 20 mSv (averaged over 5 years)
D) 100 mSv per year

- In computed radiography (CR), the imaging plate is made of:
A) Silver halide
B) Photostimulable phosphor
C) Cesium iodide
D) Selenium

- The image matrix size affects:
A) Exposure time
B) Image resolution
C) kVp
D) Focal spot size

- Ethical approval for radiological research involving patients is provided by:
A) Radiation Safety Officer
B) Ethics Committee / Institutional Review Board
C) Principal Investigator
D) AIIMS

- The lead equivalence of a standard protective apron is:
A) 0.1 mm Pb
B) 0.25 mm Pb
C) 0.5 mm Pb
D) 1.0 mm Pb

- MRI safety excludes patients with:
A) Titanium implants
B) Pacemakers
C) Dental fillings
D) Hip replacements

- The best imaging modality for detecting microcalcifications in breast tissue is:
A) MRI
B) Mammography
C) Ultrasound
D) CT

- The Gafchromic film is used for:
A) MRI contrast
B) Dosimetry
C) Image display
D) Bone densitometry

- The principle of SPECT imaging is based on:
A) Transmission of gamma rays
B) Emission of gamma photons from radionuclides
C) Attenuation correction
D) Photoelectric absorption

- Dual-energy CT is mainly used for:
A) Bone density measurement
B) Tissue characterization and material differentiation
C) Low-dose imaging
D) MR-compatible scans

- Artificial intelligence in radiology is primarily used for:
A) Equipment repair
B) Automated image interpretation
C) Patient scheduling
D) Dosimeter calibration

- The term “signal-to-noise ratio” (SNR) in imaging refers to:
A) Radiation dose
B) Image sharpness
C) Image quality vs noise
D) Exposure index

- Which of the following is NOT a component of an X-ray tube?

a) Cathode

b) Anode

c) Collimator

d) Gradient coil

22. Which material is commonly used as a contrast agent in X-ray imaging?

a) Gadolinium

b) Barium sulfate

c) Technetium-99m

d) Iodine-131

23. Who developed the first CT scanner?

a) Raymond Damadian

b) Allan Cormack

c) Godfrey Hounsfield

d) Paul Lauterbur

24. What is the typical frequency range for diagnostic ultrasound?

a) 1–10 Hz

b) 2–20 MHz

c) 50–100 kHz

d) 1–10 GHz

25. Which of the following devices is used to monitor radiation exposure in radiology personnel?

- Dosimeter (TLD/film badge)

- CT scanner

- Lead gloves

- Contrast injector