|  | # FACULTY OF _____________________________ # DEPARTMENT OF __ELECTRICAL AND ELECTRONICS ENGINEERING__ |
| --- | --- |

A

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Research Methodology

(25×1=25)

- The question starts here

- Option A

- Option B

- Option C

- Option D

- The question starts here

- Option A

- Option B

- Option C

- Option D

- The question starts here

- Option A

- Option B

- Option C

- Option D

- The question starts here

- Option A

- Option B

- Option C

- Option D

Subject-Specific Questions

(25×1=25)

- A series RLC circuit has R = 10 Ω, L = 0.1 H and C = 10µF. If a 100V DC source is applied at    t = 0, the system is

- Critically damped

- Overdamped

- Undamped

- Underdamped

- In a series RLC circuit at resonance, if L is increased and C is decreased such that the resonance frequency remains constant, the Q-factor

- Increases

- Decreases

- Remains constant

- Becomes zero

- A 24 V DC source with an internal resistance of 6 Ω is connected to a variable load resistor. What is the maximum power that can be transferred to the load?

  - 96W

  - 48W

  - 24W

  - 12W

- In a balanced three-phase, 4-wire, star-connected load, if the impedance of the R-phase is short-circuited while the other phases remain balanced, the neutral current will become

  - Zero

  - Equal to the original line current

  - √3 times the original line current

  - Dependent on the phase sequence

- Which of the following is NOT true for a delta-connected, unbalanced load?

  - Line voltage equals phase voltage

  - Line currents are vector sums of phase currents

  - The sum of line currents is zero

  - The sum of phase currents is zero

- In a DC series motor, the torque developed is 𝑇1 at a current 𝐼. If the current is doubled (2𝐼) while the motor is operating below magnetic saturation, the torque 𝑇2 becomes

  - 𝑇1

  - 2𝑇1

  - 3𝑇1

  - 4𝑇1

- If a 50 Hz transformer is operated at 60 Hz while the maximum flux density is kept constant, what happens to the voltage rating and hysteresis loss?

  - Voltage increases, Hysteresis loss increases

  - Voltage increases, Hysteresis loss decreases

  - Voltage decreases, Hysteresis loss increases

  - Voltage decreases, Hysteresis loss decreases

- A 3-phase induction motor develops maximum torque at a slip of 0.8 with no external rotor resistance. If the rotor resistance is doubled, the new slip for maximum torque is

  - 0.4

  - 0.8

  - 1.2

  - 1.6

- A synchronous motor is running at leading power factor. If the load on the shaft is reduced, the power angle will decrease, and the armature current will

  - Become more leading

  - Become more lagging

  - Remain at the same leading

  - Become unity

- A magnetic relay is excited by a constant voltage source. If the air gap is reduced, the co-energy ________ and the field energy ________.

  - Increases, Decreases

  - Decreases, Increases

  - Increases, Increases

  - Decreases, Decreases

- The “Latch-up” phenomenon in an IGBT is caused by

  - Turning ON of the parasitic NPN transistor

  - Turning ON of the parasitic SCR

  - Gate oxide breakdown due to high dv/dt

  - Excessive minority carrier storage in the drift region

- A 3-phase semi-converter feeds a highly inductive load. If the firing angle 𝛼>60∘, the output current conduction becomes

  - Continuous

  - Discontinuous

  - Cannot be determined

  - Continuous, but with high ripples

- A boost converter operating in Discontinuous Conduction Mode exhibits a transfer function that is dependent on

  - Duty cycle only

  - Input voltage and load resistance

  - Duty cycle and input voltage

  - Duty cycle and load resistance

- Consider a single-phase Full Bridge Voltage Source Inverter (VSI) supplying an R–L load using PWM control. If the switching frequency is increased, the amplitude of the first dominant harmonic will

  - Decrease

  - Increase

  - Remain constant

  - Shift to a lower frequency

- What is the main advantage of Integral Cycle Control over Phase Angle Control?

  - Higher efficiency

  - Higher output voltage range

  - Zero voltage switching

  - Faster response time

- Which of the following relay types is most suitable for protecting long transmission lines?

  - Impedance relay

  - Reactance relay

  - Overcurrent relay

  - Mho’s relay

- For a given transmission line, if the spacing between conductors is increased by 20%, the inductance and capacitance respectively will

  - Increase, Decrease

  - Decrease, Increase

  - Increase, Increase

  - Decrease, Decrease

- Which type of thermistor is typically used for over-current protection?

  - Thermopile

  - Zero Temperature Coefficient

  - NTC (Negative Temperature Coefficient)

  - PTC (Positive Temperature Coefficient)

- In a 4-quadrant chopper-fed DC series motor drive, if the motor is operating in regenerative braking in reverse direction, which quadrant is active?

  - Quadrant I

  - Quadrant II

  - Quadrant III

  - Quadrant IV

- In an HVDC link, protection against overvoltages on the DC side is primarily achieved by

  - DC circuit breakers

  - Bypass valves

  - Surge arresters

  - Firing angle control

- In a Bode plot, the slope of a system at very high frequencies is −60 dB/decade. If the system has 2 zeros, how many poles must it have?

  - 2

  - 3

  - 4

  - 5

- A system with state matrix 𝐴 is asymptotically stable if all its eigenvalues lie in the

  - Left-half of the s-plane

  - Right-half of the s-plane

  - Inside the unit circle in the z-plane

  - Left-half of the s-plane, provided it is controllable

- What is the effect of adding a pole to a system's loop transfer function in the root locus?

  - It decreases the damping ratio

  - It increases the damping ratio

  - It increases stability

  - It decreases stability

- For a power system with a single machine connected to an infinite bus, the transient stability limit is ____ the steady-state stability limit.

- Lower than

- Higher than

- Equal to

- Independent of

- The damping method used in moving iron instruments is generally

- Eddy current damping

- Fluid friction damping

- Air friction damping

- Electrostatic damping

Answer Key

A

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 | (Example:) A | 26 | D |
| 2 |  | 27 | A |
| 3 |  | 28 | C |
| 4 |  | 29 | C |
| 5 |  | 30 | D |
| 6 |  | 31 | D |
| 7 |  | 32 | A |
| 8 |  | 33 | D |
| 9 |  | 34 | A |
| 10 |  | 35 | C |
| 11 |  | 36 | B |
| 12 |  | 37 | B |
| 13 |  | 38 | D |
| 14 |  | 39 | A |
| 15 |  | 40 | C |
| 16 |  | 41 | D |
| 17 |  | 42 | A |
| 18 |  | 43 | D |
| 19 |  | 44 | D |
| 20 |  | 45 | C |
| 21 |  | 46 | D |
| 22 |  | 47 | A |
| 23 |  | 48 | D |
| 24 |  | 49 | A |
| 25 |  | 50 | C |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.