|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING |
| --- | --- |

A

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : EEE |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- An infinite ladder network is constructed with series resistors R1 and shunt resistors R2. It has a characteristic equivalent resistance Req at its input terminals. If a single shunt resistor R2, located very far from the input, is replaced by an open circuit, what is the approximate change in Req?

- Req increases significantly

- Req approaches zero

- The change is zero

- Req approaches infinity

- A complex linear network is represented by its Norton equivalent, consisting of a 5 A current source in parallel with a 2 Ω Norton resistance. If the output terminals of this network are open-circuited, what will be the potential difference across the terminals?

  - 10 V

  - 5 V

  - 2.5 V

  - 0 V

- An ideal series LC circuit, initially at rest, is connected to a DC source Vdc at t = 0. What is the maximum voltage that will ever appear across the capacitor?

  - Vdc

  - 0

  - Vdc / 2

  - 2 Vdc

- When measuring power in a 3-phase, 3-wire, balanced system using the two-wattmeter method, one of the wattmeters W1 reads a positive value, while the second wattmeter W2 reads exactly zero. What does this conceptually imply about the load?

- The load's power factor is 1.0

- The load's power factor is 0.866 (lagging or leading)

- The load's power factor is 0

- The load's power factor is 0.5 (lagging or leading)

- A series RLC circuit has a resonant frequency f and a bandwidth BW. The capacitor C is then replaced with a new capacitor Cnew = C/4, while R and L remain unchanged. Which of the following correctly represents the new resonant frequency fnew and the new bandwidth BWnew?

  - fnew = f    and     BWnew = 2BW

  - fnew = 2f    and    BWnew = 2BW

  - fnew = 2f    and   BWnew = BW

  - fnew = f     and     BWnew = BW

- The primary, fundamental function of compensating windings in a large DC machine is to

- Neutralize the armature reaction MMF directly under the main pole faces

- Increase the main field flux to compensate for IaRa drop

- Neutralize the armature reaction MMF in the interpolar region

- Neutralize the reactance voltage induced in the commutating coil

- Compared to a 50 Hz power transformer of the same kVA rating, a 1 kHz audio frequency transformer would require

  - A much larger iron core

  - Significantly thicker laminations

  - A much smaller iron core

  - A core made of solid (non-laminated) steel

- Regenerative braking occurs in an induction motor when

- Rotor speed exceeds synchronous speed

- Rotor speed is less than synchronous speed

- The slip is greater than unity

- The supply phase sequence is reversed

- If a synchronous generator runs at constant mechanical speed but its excitation is suddenly increased, then

- Output power increases instantly

- Torque angle decreases momentarily

- Frequency increases

- Reactive power output increases

- For a continuous average torque to be produced in any ideal rotating electrical machine, which of the following conditions regarding the stator MMF and rotor MMF is essential?

- The stator MMF must be stationary, and the rotor MMF must rotate

- The stator MMF and rotor MMF must be stationary with respect to each other

- The stator MMF and rotor MMF must rotate at different speeds to create slip

- The stator MMF and rotor MMF must be perfectly aligned

- In a buck-boost converter operating in Continuous Conduction Mode, the average inductor voltage over one period must be zero because

- The inductor behaves as a short circuit to DC in steady state

- It ensures current continuity through the diode

- Otherwise energy would accumulate in the inductor indefinitely

- The converter’s duty ratio remains fixed during each cycle

- The main reason IGBTs exhibit a latch-up problem is

- Excessive Miller capacitance

- Activation of the parasitic thyristor structure within the device

- Regenerative feedback between internal PNP and NPN transistor sections

- Punch-through in the drift region

- In a single-phase fully controlled converter supplying an R–L load, the boundary between continuous and discontinuous conduction is reached when

- The load current becomes zero before the next pair of thyristors is triggered

- The firing angle is equal to zero degrees

- The load inductance is large enough to maintain constant current irrespective of firing angle

- The current ripple in the load is at its minimum value

- In a three-phase voltage source inverter operating with 180° conduction mode, at any instant of time

- Only one switch conducts, corresponding to the phase with maximum voltage

- Two switches conduct simultaneously, each from a different leg

- One switch in each leg conducts, forming a current path for all three phases

- All six switches conduct alternately to maintain symmetry

- In a single-phase AC voltage controller with resistive load, if the firing angle α is increased, the RMS output voltage

- Decreases with α

- Increases with α

- First decreases, then increases with α

- First increases, then decreases with α

- Replacing a single conductor with a bundled conductor configuration in a high voltage transmission line primarily aims to

- Reduce the mechanical strength of towers

- Increase shunt capacitance for stability

- Decrease the surge impedance of the line

- Reduce corona loss and radio interference

- In the design of a power distribution feeder, the conductor size is selected through a multi-step process. Which of the following properties establishes the minimum required size of the conductor to ensure operational safety and prevent thermal damage?

- Allowable voltage drop

- Current-carrying capacity

- System power factor

- Conductor impedance

- In a medium transmission line, if the line is represented by a nominal π model, neglecting shunt capacitance will cause

- Overestimation of receiving-end voltage

- Accurate prediction of Ferranti effect

- Over estimation of reactive power flow

- Underestimation of voltage regulation

- The application of an instantaneous overcurrent relay as a primary protection element is most effective and selective when applied to

- Long transmission lines

- Short lines with low fault levels

- Short lines with high fault currents

- Backup protection of transformers

- In a high-voltage underground cable using capacitance grading, the relative permittivity of the concentric dielectric layers is arranged to

- Decreasing dielectric constants from core to sheath

- Increasing dielectric constants from core to sheath

- Equal dielectric constants

- No specific dielectric arrangement

- The operational principle of a Hall effect sensor relies on the generation of a transverse Hall voltage. The magnitude of this voltage is primarily used as a direct measure of

- Electrostatic field strength

- Magnetic flux density

- Charge carrier mobility

- Electromagnetic wave frequency

- Adding a zero to the right-half-plane of an open-loop transfer function is known to have what two "non-minimum phase" effects?

- It adds phase lead and causes initial undershoot in the step response

- It adds phase lead and significantly increases the steady-state error

- It adds phase lag and causes initial undershoot in the step response

- It adds phase lag and eliminates the steady-state error

- A common modification to the standard PID algorithm is to apply the derivative action only to the process variable rather than the error signal. What phenomenon does this 'Derivative on Measurement' configuration primarily mitigate?

- Integral windup during actuator saturation

- Amplification of high-frequency sensor noise

- A large, sudden spike in the controller output

- Poor phase margin when the integral gain is high

- In the analysis of a linear time-invariant system, the closed-loop transfer function is found to have simple, non-repeated poles located exactly on the imaginary axis of the s-plane. All other poles are strictly in the left-half plane. This system is classified as

- Marginally stable

- Asymptotically stable

- Unstable

- Conditionally stable

- In a standard second-order control system, the maximum percent overshoot observed in the step response depends on

- Only the natural frequency

- Only the damping ratio

- Both the damping ratio and the natural frequency

- Neither damping ratio nor the natural frequency