|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF CIVIL ENGINEERING |
| --- | --- |

B

Ph.D. Entrance Examination – July 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : Civil Engineering |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

Section B: Subject-Specific Core Topics (25 Questions | 1 Mark Each)

1. A cement bag comprises of 0.035 m3 by volume. How many bags will one tonne of cement will comprise?

- 16

- 18

- 17

- 20

2. The total angle of deflection of a transition curve is:

- Spiral angle

- Spiral angle/3

- Spiral angle/2

- Spiral angle/4

3. Average rate of water consumption per head per day as per Indian standard is :

  - 100 liters

  - 165 liters

  - 135 liters

  - 200 liters

4. The reduction coefficient of reinforced concrete column with an effective length of 4.8 m and size 250 mm×300 mm is

- 0.80

- 0.85

- 0.90

- 0.95

5. The F/M ratio is controlled in the activated sludge process by adjusting the

- flowrate

- volume of aeration tank

- amount of sludge wasting

- BOD

6. A suspension bridge with a two-hinged stiffening girder is

- statically determinate

- indeterminate of one degree

- indeterminate of two degrees

- a mechanism

7. Penetration test on bitumen is used for determining its:

  - Grade

  - Ductility

  - Viscosity

  - Temperature susceptibility

8. A fixed beam AB is subjected to a triangular load varying from zero at end A to w per unit length at end B. The ration of fixed end moment at B to A will be

- 1/2

- 1/3

- 2/3

- 3/2

9. Web buckling occurs in a beam due to excessive

- Direct tensile stress in the web

- Bending tensile stress in the web

- Torsional shear stress in the web

- Compressive stress in the web

10. Consider the following statements:

a. Quick condition and liquefaction of saturated sands are based on similar phenomenon.

b. Quick condition is associated with only earth dams.

c. Liquefaction is possible in dry sand also

d. Liquefaction is associated with increase in pore water pressure due to vibrations.

Which of these statements are corrects?

- 2 and 4

- 1 and 4

- 1and2

- 1,3 and 4

11. Battens provided for a compression member shall be designed to carry a transverse shear equal to

- 2.5% of axial force in the member

- 5% of axial force in the member

- 10% of axial force in the member

- 20% of axial force in the member

12. A field measure 40 hectares. When 8 cumecs of water was supplied for 6 hours, 30 cm of water was stored in the root zone. The field application efficiency is nearly

- 70%

- 80%

- 85%

- 90%

13. Basic working principle of operation of a GPS device is :

  - Trilateration

  - Intersection

  - Triangulation

  - Resection

14. A footing of concrete of size 1.5 x 1 x 0.6 m is placed on a dense saturated sand, which lied below the ground water table whose saturated unit weight is 20 kN/m3. The maximum pressure that can be applied with a factor of safety of 3 through the concrete footing, ensuring no bearing capacity failure in soil using Terzaghi’s bearing capacity equation without considering the shape factor, depth factor and Inclination factor [Assume Nq= 350, C= 0, Nγ= 40, γw= 10 kN/m3]

- 26. 66 Kpa

- 66.66 Kpa

- 80Kpa

- 120Kpa

15. A steel rod of 16 mm diameter has been used as a tie in a bracing system, but may be subject to possible reversal of stress due to the wind. What is the maximum permitted length of the member?

- 1600 mm

- 1400 mm

- 1200 mm

- 1000 mm

16. The carryover factor in a prismatic member whose far end is hinged is

- 0

- ½

- ¾

- 1

17. In case of a simply supported rectangular beam of span L and loaded with a central load W, the length of elasto-plastic zone of the plastic hinge, is

- L/2

- L/3

- L/4

- L/5

18. An increase in temperature on the top fibre of a simply supported beam will cause

- Downward deflection

- Upward deflection

- No deflection

- Angular rotation about neutral axis

19. A two hinged semi-circular arch of radius R carries a concentrated load W at the crown. The horizontal thrust is

- 𝑊 2𝜋

- 𝑊 𝜋

- 2𝑊 3𝜋

- 4W/3

20. A two span continuous beam ABC is simply supported at A and C and is continuous over support B. Span AB = 6 m, BC = 6m. The beam carries a UDL of 2 t/m over both the spans. B is constant for the entire beam. The fixed end moment at B in span BA or BC would be

- 12 t.m

- 8 t.m

- 9 t.m

- 6 t.m

21. If a simply supported concrete beam, prestressed with a forced of 2500 kN is designed by load balancing concept for an effective span of 10m and to carry a total load of 40 kN/m, the central dip of the cable profile should be

- 100 mm

- 200 mm

- 300 mm

- 400 mm

22. In a concrete pavement, during summer at and soon after mid-day, the combined stress at the interior of the slab is equal to

- Wheel load stress + Temperature warping stress + Sub grade resistant stress

- Wheel load stress + Temperature warping stress Sub grade resistant stress

- Wheel load stress- Temperature warping stress + Sub grade resistant stress

- Wheel load stress- Temperature warping stress Sub grade resistant stress

23. The Castigliano's second theorem can be used to compute deflections

- in statically determinate structures only

- for any type of structure

- at the point under the load only

- for beams and frames only

24. As per IS code, the maximum longitudinal pitch allowed in bolted joints of tension members is nominally

- 12 times the thickness of the plate

- 12 times the diameter of the bolt

- 16 times the thickness of the plate

- 16 times the diameter of the bolt

25. The primary compression failure in R.C.C. beam is caused in

- under-reinforced beams

- balanced beams

- Over-reinforced beams

- all types of beams.