|  | # College of Allied Health Sciences, SRMIST, TRC # Subject: Medical Imaging Technology |
| --- | --- |

A

Ph.D. Entrance Examination – November 2025

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MIT |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- Grid ratio is defined as:
A) Height of lead strips to their width
B) Height of lead strips to interspace width
C) Width of lead strips to height
D) None of the above

- Bremsstrahlung radiation occurs when:
A) Electrons hit the target atoms directly
B) Electrons are deflected by the nucleus
C) Electrons combine with protons
D) X-rays interact with outer shell electrons

- The penetrating power of an X-ray beam is primarily determined by:
A) mA
B) Exposure time
C) kVp
D) Filtration

- The function of rectifiers in an X-ray circuit is to:
A) Step up voltage
B) Step down voltage
C) Convert AC to DC
D) Regulate filament current

- The half-value layer (HVL) of an X-ray beam is a measure of:
A) Beam intensity
B) Beam quality
C) Beam quantity
D) Tube efficiency

- The focusing cup in an X-ray tube is made of:
A) Lead
B) Copper
C) Molybdenum
D) Nickel

- The main advantage of three-phase generators over single-phase generators is:
A) Reduced patient dose
B) Increased ripple factor
C) Lower exposure time
D) Better contrast

- The material commonly used for the rotating anode target is:
A) Tungsten
B) Lead
C) Silver
D) Aluminum

- The anode angle in a general diagnostic X-ray tube typically ranges between:
A) 1–5°
B) 6–20°
C) 25–45°
D) 50–70°

- In a chest radiograph, the patient’s inspiration is adequate if:
A) 5 ribs are visible above the diaphragm
B) 6 ribs are visible
C) 8–10 posterior ribs are visible
D) 4 ribs are visible

- The inverse square law applies to:
A) X-ray beam intensity
B) Tube current
C) Beam filtration
D) Image sharpness

- Barium sulfate is contraindicated in cases of:
A) Hiatus hernia
B) Intestinal obstruction
C) Esophageal reflux
D) Perforation of GI tract

- The best projection for visualizing the maxillary sinuses is:
A) Caldwell’s
B) Waters’
C) Towne’s
D) Lateral

- The standard source-to-image distance (SID) in general radiography is:
A) 60 cm
B) 90 cm
C) 100 cm
D) 180 cm

- The imaging technique most suitable for soft tissue evaluation is:
A) X-ray
B) CT
C) MRI
D) Ultrasonography

- The safest contrast media for intravenous use is:
A) High-osmolar ionic contrast
B) Non-ionic iodinated contrast
C) Barium sulfate
D) Gadolinium chelate

- For a lateral skull projection, the central ray passes through:
A) Nasion
B) External auditory meatus
C) Glabella
D) Acanthion

- A radiograph that is too dark may be due to:
A) Low kVp
B) Underexposure
C) Overexposure
D) Insufficient development

- The most common cause of motion blur in radiographs is:
A) Long exposure time
B) High kVp
C) Small focal spot
D) Increased SID

- The “heel effect” in X-ray imaging is caused by:
A) Beam filtration
B) Line focus principle
C) Anode-cathode geometry
D) Electron backscatter

- In ultrasound imaging, the piezoelectric crystal converts:
A) Electrical energy to magnetic energy
B) Electrical energy to sound waves
C) Sound waves to light
D) Mechanical energy to electric current only

- Hounsfield unit (HU) for water is:
A) –1000
B) 0
C) +1000
D) +2000

- The basic physical principle behind CT imaging is:
A) Reflection of sound waves
B) Attenuation of X-rays
C) Magnetic resonance
D) Radioisotope decay

- Doppler ultrasound is primarily used for assessing:
A) Bone density
B) Blood flow
C) Airway structures
D) Muscle composition

- MRI signal depends mainly on:
A) Atomic number
B) Proton density and relaxation times
C) Electron density
D) X-ray attenuation