|  | # FACULTY OF # ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING |
| --- | --- |

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | A : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (Name, Application Number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Research Methodology

(25×1=25)

Subject-Specific Questions

(25×1=25)

26. A D flip-flop has D = 1 and clock rising edge occurs. The output Q becomes:

A. 0

B. 1

C. Toggles

D. Unchanged

27. Which statement about Schottky diodes is CORRECT?

A. Higher forward voltage drops than Si PN junction

B. Minority carrier device — slow switching

C. Majority carrier device — fast switching

D. Same forward voltage as silicon PN junction

28. Which logic gate outputs a HIGH only when all inputs are LOW?

A. AND

B. OR

C. NAND

D. NOR

29. A 4-bit binary ripple counter has a clock of 16 kHz. The output frequency at the MSB (Q3) is:

A. 16 kHz

B. 8 kHz

C. 2 kHz

D. 1 kHz

30. An NPN BJT with β = 100, IB = 20μA. The collector current IC is:

A. 0.2 mA

B. 2 mA

C. 20 mA

D. 200 μA

31. A bulb controlled by two switches on different floors, where either switch can turn the bulb ON or OFF independently, corresponds to which logic gate?

A. AND

B. OR

C. XOR

D. NAND

32. Thevenin’s theorem states that any linear two-terminal network can be replaced by:

A. A voltage source in series with a resistor

B. A current source in parallel with a resistor

C. A voltage source in parallel with a resistor

D. A current source in series with a resistor

33. Bayes theorem is used to calculate:

A. Joint probability

B. Marginal probability

C. Conditional probability

D. Independent probability

34. An AM signal has carrier amplitude Ac = 10V, message amplitude Am = 6V. Modulation index μ is:

A. 10/6

B. 0.6

C. 1.0

D. 0.4

35. Which of the following is a voltage-controlled device?

A. SCR

B. FET

C. BJT

D. Diode

36. The cut-in voltage of a Germanium diode is:

A. 0.7 V

B. 0.3 V

C. 0.5 V

D. 1.0 V

37. If a silicon diode is forward biased in a circuit with 12 V supply and 240 Ω resistor, what does the voltage drop across the diode?

A. 0.3 V

B. 0.7 V

C. 1.2 V

D. 5 V

38. Which of the following is a trivalent doping element?

A. Phosphorus

B. Boron

C. Antimony

D. Arsenic

39. For normal active operation of an NPN transistor, the emitter-base junction must be:

A. Forward biased

B. Reverse biased

C. Either forward or reverse biased

D. No bias required

40. A message signal of peak value 2 V is used to modulate a carrier wave of peak value 10 V. The modulation index of the amplitude modulated wave is:

A. 0.2

B. 5

C. 0.5

D. 0

41. The bit rate of a digital communication system is R kbit/s and the modulation used is 32-QAM. The minimum bandwidth required for ISI-free transmission is:

A. R/10 Hz

B. R/10 kHz

C. R/5 Hz

D. R/5 kHz

42. An FM signal has maximum frequency deviation Δf = 75 kHz, message bandwidth W = 15 kHz. The modulation index β is:

A. 15

B. 0.2

C. 90

D. 5

43. The variation of frequency of a carrier wave with respect to the amplitude of the modulating signal is called:

A. Amplitude modulation

B. Frequency modulation

C. Phase modulation

D. Pulse width modulation

44. The convolution of two signals in time domain corresponds to which operation in frequency domain?

A. Addition

B. Multiplication

C. Differentiation

D. Integration

45. A system is said to be causal if its output depends on:

A. Future inputs only

B. Past and present inputs only

C. Present inputs only

D. Past, present, and future inputs

46. S-parameters in transmission lines are used to:

A. Describe power reflection and transmission at ports

B. Represent voltage and current

C. Calculate resistance only

D. Measure capacitance

47. The Poynting vector represents:

A. Magnetic field intensity

B. Electric field intensity

C. Power flow per unit area

D. Charge density

48. A signal has maximum frequency 4 kHz. The minimum Nyquist sampling rate is:

A. 4 kHz

B. 1 kHz

C. 8 kHz

D. 16 kHz

49. FM signal: s(t) = 10cos(2π×10⁶t + 5sin(2π×10³t)). The modulation index β is:

A. 2.5

B. 5

C. 10

D. 15

50. A (7,4) Hamming code has minimum Hamming distance dmin = 3. It can:

A. Detect 1 error, correct 0 errors

B. Detect 3 errors, correct 1 error

C. Detect 2 errors, correct 1 error

D. Detect 1 error, correct 1 error

Answer Key

A

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 |  | 26 | B |
| 2 |  | 27 | C |
| 3 |  | 28 | D |
| 4 |  | 29 | D |
| 5 |  | 30 | B |
| 6 |  | 31 | C |
| 7 |  | 32 | A |
| 8 |  | 33 | C |
| 9 |  | 34 | B |
| 10 |  | 35 | B |
| 11 |  | 36 | B |
| 12 |  | 37 | B |
| 13 |  | 38 | B |
| 14 |  | 39 | A |
| 15 |  | 40 | A |
| 16 |  | 41 | D |
| 17 |  | 42 | D |
| 18 |  | 43 | B |
| 19 |  | 44 | B |
| 20 |  | 45 | B |
| 21 |  | 46 | A |
| 22 |  | 47 | C |
| 23 |  | 48 | C |
| 24 |  | 49 | B |
| 25 |  | 50 | C |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.