|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # SCHOOL OF MECHANICAL ENGINEERING |
| --- | --- |

A

Ph.D. Entrance Examination – June 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MECHANICAL |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- Big refrigeration plants (of capacity of the order of 1000 tons) usually employ following type of refrigeration plant

(A) vapour absorption

(B) vapour compression

(C) air cycle

(D) electrolux refrigerator

- Wet bulb temperature is
	(A) indication of amount of moisture in air
	(B) measured by wetting the bulb of the thermometer
	(C) less than dry-bulb temperature
	(D) dependent on the dryness and temperature of air

- The purpose of sprue down is to

(A) feed the casting at a rate consistent with the rate of solidification

(B) act as a reservoir for molten metal

(C) help feed the casting until all solidification takes place

(D) feed molten metal from pouring basin to gate

- Injection moulding is the ideal method of processing

(A) plastics

(B) thermo-setting plastics

(C) thermoplastics

(D) non-ferrous materials

- On volume basis, air contains following parts of oxygen
(A) 21
(B) 23
(C) 25
(D) 77

- A perfect gas at 27°C was heated until its volume was doubled. The temperature of the gas will now be
(A) 270°C
(B) 540°C
(C) 327°C
(D) 729°C

- A particle moves along a straight line such that distance (x) traversed in t seconds is given by x=t2(t−4), the acceleration of the particle will be given by the equation

(A) 3t2−2t

(B) 3t2+2t

(C) 6t−8

(D) 6t−4

- When P is the effort and W is the load then linear law of machines is given by the relation

(A) W=mP+c

(B) W=mP−c

(C) P=mW−c

(D) P=mW+c

where m and c are constants

- Which is incorrect statement about results of cold working?

(A) it increases corrosion resistance

(B) it increases strength, elasticity and hardness with a corresponding decrease in ductility

(C) handling of parts is easier

(D) dimension tolerances and finish are high

- Tool life is most affected by machine

(A) cutting speed

(B) tool geometry

(C) feed and depth

(D) microstructure of material being cut

- A liquid jet from a nozzle exposed to atmosphere traverses along

(A) a straight line	(B) a circular path

(C) an elliptical path	(D) parabolic path

- The included angle for triangular notch for maximum discharge is

(A) 45o	(B) 60o

(C) 90o	(D) 108o

- Turbofan engine employs

(A) one air stream

(B) two or more air streams

(C) no air stream

(D) solid fuel firing

- Pick up the wrong statement about water tube boiler in comparison to fire tube boilers
	(A) former generates steam at high pressure
	(B) former occupies less space for same power
	(C) rate of steam flow is more in former case
	(D) chances of explosion are less in former case.

- The specific fuel consumption per BHP hour for a petrol engine is approximately

(A) 0.15 kg
	(B) 0.2 kg
	(C) 0.25 kg
	(D) 0.3 kg

- Which of the following refrigerants has highest refrigeration efficiency?

(A) Freon-11

(B) Freon-22

(C) R-12

(D) R-113

- The resultant of the following three couples

20 kg force, 0.5 m arm, + ve sense

30 kg force, 1 m arm, – ve sense

40 kg force, 0.25 m arm, + ve sense

having arm of 0.5 m will be

(A) 20 kg, – ve sense

(B) 20 kg, + ve sense

(C) 10 kg, + ve sense

(D) 10 kg, – ve sense

- Two bodies of 100 kg and 400 kg are resting on two inclined planes α and β towards each other and the bodies are joined together by a string passing over a pulley connected at the top of inclined planes. Coefficient of friction for two bodies with their inclined planes are μ₁ and μ₂. Tension in string will be

(A) 100 kg

(B) 300 kg

(C) 400 kg

(D) 500 kg

- Pressure ratio in gas turbines is the ratio of

(A) compressor pressure ratio

(B) highest pressure to exhaust pressure

(C) inlet pressure to exhaust pressure

(D) pressures across the turbine

- In regenerative air preheater, the heat is transferred

(A) from a metal wall from one medium to another

(B) from heating an intermediate material and then heating the air from this material

(C) by direct mixing

(D) heat is transferred by bleeding some gas from furnace

- When two springs are in series (having stiffness K), the equivalent stiffness will be

(A) K

(B) K/2

(C) 2K

(D) K/4

- The maximum efficiency of a screw jack having square threads and friction angle of 30° will be

(A) 11%

(B) 20%

(C) 30%

(D) 33%

- If a certain amount of dry ice is mixed with same amount of water at 80°C, the final temperature of mixture will be

(A) 80o C

(B) 0o C

(C) 40o C

(D)  20o C

- 2 kg of substance receives 500 kJ and undergoes a temperature change from 100°C to 200°C. The average specific heat of substance during the process will be

(A) 5 KJ / Kg.K

(B) 2.5 KJ / Kg.K

(C) 10 KJ / Kg.K

(D) 25 KJ/Kg.K

25. In convection heat transfer from hot flue gases to water tube, even though flow may be turbulent, a laminar flow region (boundary layer of film) exists close to the tube. The heat transfer through this film takes place by
(A) convection
(B) radiation
(C) conduction
(D) both convection and conduction