|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # SCHOOL OF MECHANICAL ENGINEERING |
| --- | --- |

B

Ph.D. Entrance Examination – June 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : MECHANICAL |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 25×1=25 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- Klein's construction can be used when

(A) crank has a uniform angular velocity

(B) crank has non-uniform velocity

(C) crank has uniform angular acceleration

(D) crank has uniform angular velocity and angular acceleration

- Fluctuation of energy of an engine is the

(A) variation of energy above and below the mean resisting torque line

(B) ratio of maximum and minimum energies

(C) difference between the maximum and minimum energies

(D) ratio of the maximum fluctuation of energy to the work done per cycle

- Bomb calorimeter is used to determine

(A) Higher calorific value at constant volume

(B) Lower calorific value at constant volume

(C) Higher calorific value at constant pressure

(D) Lower calorific value at constant pressure

- Most serious energy loss in impulse steam turbine is

(A) steam leakage between diaphragms and shaft

(B) leakage over blade ends through radial clearance passages

(C) frictional resistance between steam and nozzle side

(D) impact loss as steam enters the moving blades

- The depth of centre of pressure in a rectangular lamina of height h with one side in the liquid surface is at

(A) h

(B)  h/3

(C) 2h/3

(D) h2

- Two balls of same material and finish have their diameters in the ratio of 2 : 1 and both are heated to same temperature and allowed to cool by radiation. Rate of cooling by big ball as compared to smaller one will be in the ratio of
(A) 1 : 1
(B) 2 : 1
(C) 1 : 2
(D) 4 : 1

- The depth of water below the spillway and after hydraulic jump are 1 m and 6 m respectively. The head lost will be

(A) 1.04 m	(B) 5 m

(C) 1.7 m	(D) 2.05 m

- The motion of a fluid is vortex if each particle of the fluid moves in a circular path with the speed which

(A) is constant

(B) is directly proportional to distance from centre

(C) varies as square of distance from centre

(D) varies inversely as the distance from centre

- Which is false statement about multistage compression?

(A) Power consumption per unit of air delivered is low

(B) Volumetric efficiency is high

(C) It is best suited for compression ratios around 7: 1

(D) The moisture in air is condensed in the intercooler

- Which of the following are the reasons for the volumetric efficiency of reciprocating compressor being less than 100%?

- Deviations from isentropic process.

- Pressure drop across the valves.

- Superheating in compressor.

- Clearance volume.

- Deviations from isothermal process.

- Leakages.

Select the correct answer from the codes given below:
Codes:
(A) 1, 2, 3 and 5
(B) 2, 3, 4 and 5
(C) 1, 4, 5 and 6
(D) 2, 3, 4 and 6.

- To transmit power from one rotating shaft to another whose axes are neither parallel nor intersecting, use

(A) spur gear

(B) spiral gear

(C) bevel gear

(D) worm gear

- In the case of flat collar bearing, the frictional force in case of uniform pressure can be assumed to be acting at

(A)  2 /3 x  (R12   - 2 R22   / (R12– R22)

(B)  2 /3 x  (R13   - 2 R23   / (R12– R22)

(C)  1/2 x (R1 + R2)

(D) 2/3 x (R1 + R2)

- Overfire burning is the phenomenon of

(A) supply of excess air

(B) supply of excess coal

(C) burning CO and unburnt in upper zone of furnace by supplying more air

(D) fuel bed firing

- The ratio of limiting friction and normal reaction is known as

(A) coefficient of friction

(B) angle of friction

(C) angle of repose

(D) sliding friction

- The ratio of diameters of Low Pressure (P1) Cylinder to High Pressure (P2) Cylinder with complete intercooling is equal to

(A) P2/P1

(B) P1/P2

(C) SQRT OF P2/P1

(D) SQRT OF P1/P2

- Which of the following is spring controlled governor?
	(A) Hartnell
	(B) Hartung
	(C) Wilson-hartnell
	(D) all of the above.

- In current carrying conductors, if the radius of the conductor is less than the critical radius, the addition of electrical insulation is desirable, as

(A) it reduces the heat loss from the conductor and thereby enables the conductor to carry a higher current.
(B) it increases the heat loss from the conductor and thereby enables the conductor to carry a higher current.
(C) it increases the thermal resistance of the insulation and thereby enables the conductor to carry a higher current.
(D) it reduces the thermal resistance of the insulation and thereby enables the conductor to carry a higher current.

- The valve rod in a steam engine is connected to an eccentric rod by

(A) cotter joint

(B) bolted joint

(C) knuckle joint

(D) universal coupling

- A grey body is one whose absorptivity
(A) varies with temperature
(B) varies with wavelength of the incident ray
(C) is equal to its emissivity
(D) does not vary with temperature and wavelength of the incident ray

- If two surfaces of area A distance L apart, of a material having thermal conductivity k are at temperatures t1 and t2, then heat flow rate through it will be

(A) KA/L  x  (t1-t2)

(B) KL/A  x  (t1-t2)

(A) K/LA  x  (t1-t2)

(A) L/KA  x  (t1-t2)

- If the compression ratio of an engine working on Otto cycle is increased from 5 to 7, the % age increase in efficiency will be

(A) 2%

(B) 4%

(C) 8%

(D) 14%

- A screw is specified by

(A) major diameter × pitch

(B) minimum diameter × length

(C) major diameter × length

(D) mean diameter × pitch

- A riveted joint may fail due to

(A) failure of the rivets

(B) failure of the plates

(C) either failure of the rivets or failure of the plates

(D) failure of both rivets or plates

- According to Avogadro's law, for a given pressure and temperature, each molecule of a gas

(A) occupies volume proportional to its molecular weight

(B) occupies volume proportional to its specific weight

(C) occupies volume inversely proportional to its molecular weight

(D) occupies same volume

- Two stainless steel foils of 0.1 mm thickness are to be joined. Which of the following processes would be best suited

(A) Gas welding

(B) Electroslag welding

(C) TIG welding

(D) MIG welding