|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ECE |
| --- | --- |

A

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : ECE |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

1. The threshold voltage of an n-channel MOSFET is 0.7 V. If VGS = 2 V and VDS = 5 V, the MOSFET operates in which region?

A.Cutoff

B. Saturation

C.  Triode

D. Breakdown

2. The voltage gain of an inverting amplifier is –100. If input voltage is 20 mV, output voltage is:

A   0.2 V

B   1 V

C    2 V

D   –2 V

3. In a silicon diode, the forward voltage drop at room temperature is approximately:

A.  0.1 V

B.  0.3 V

C.  0.7 V

D.  1.1 V

4. For a CE amplifier, increasing emitter resistance RE:

- A. Increases gain

- B. Decreases gain

- C. Does not affect gain

- D. Causes distortion

5. The Boolean expression AB + A'B simplifies to:

A. A + B

B. B

C. A

D. AB'

6. A 400 watt carrier is modulated to a depth of 75 percent.Calculate the total power in the AM modulated wave.

- 512.5W

- 250W

- 400.5W

- 250.5W

7. The antenna current of an AM broadcast transmitter, modulated to a depth of 40 percent by an audio sine wave is 11 A.It increases to 12 A as a result of simultaneous modulation by another audio sine wave. What is the modulation index due to this second wave.

- 0.643

- 0.757

- 0.123

- 0.346

8. A certain transmitter radiates 9 kW with the carrier unmodulated, and 10.125 kW when the carrier sinusoidally modulated using DSB-AM.Calculate the modulation index.

- 0.5

- 0.3

- 0.4

- 0.1

9. A broadcasting station has transmitter   power of 10.125 kW when it is modulated by sine wave with modulation index 0.5.If another sine wave is simultaneously transmitted with modulation index 0.4,what is the effective modulation index.

A.       0.64

B.       0.5

C.       0.4

D.      0.45

10. A DSBSC transmitter radiates 1 kW when the modulating percentage is 60%.How much of carrier power is required if we want to transmit the same message by an AM transmitter?

- 5.56 kW

- 4.56 kW

- 3.23 kW

- 7.12 kW

11. The number of flip-flops required to design a mod-16 counter is:

A.  3

B.  4

C.  5

D. 16

12. A system with transfer function 10/(s(s+2)) is:

A. Type 0

B. Type 1

C. Type 2

D. Unstable

13. Increasing the gain of a feedback system generally:

A. Decreases bandwidth

B. Increases steady-state error

C. Increases bandwidth

D. Decreases damping

14. In an RLC circuit at resonance:

A. Impedance is maximum

B. Current is minimum

C. Voltage across L and C are equal and opposite

D. Power factor < 1

15. The impedance of a 10 Ω resistor in series with a 10 Ω inductive reactance is:

A. 10 Ω

B. 14.14 Ω

C. 20 Ω

D. 0 Ω

16. Find the carrier  frequency of PM wave represented by the voltage equation

- 95.5 MHz

- 6000 kHz

- 1000 kHz

- 12 MHz

17.   In commercial TV transmission in India, picture and speech signals are modulated respectively (Picture) (Speech)

A.  VSB and VSB

B.  VSB and SSB

C.  VSB and FM

D.  FM and VSB

18. For a message signal mt=cos⁡(2πfmt) and carrier of frequency fc , Which of the following represents a single side-band (SSB) signal?

A.  cos2πfmt cos⁡(2πfct)

B. cos⁡(2πfct)

C. cos[2𝜋(fc + fm)𝑡]

D.  [1+cos2πfmt]. cos2πfct

19. In binary data transmission DPSK is preferred to PSK because

- A coherent carrier is not required to be generated at the receiver

- For a given energy per bit, the probability of error is less

- The 180  phase shifts of the carrier are unimportant

- More  protection is provided against impulse noise

20. If modulating signal frequency is 5 KHz, What would be the sampling period?

- 10  KHz

- 5 KHz

- 6 MHz

- 8  KHz

21. In delta modulation, the granular noise  can be reduced by

- decreasing the step size

- decreasing the granular noise

- decreasing the sampling noise

- increasing the step size

22. A band limited signal is sampled at the Nyquist rate. The signal can be recovered by passing the samples through

- An LC filter

- PLL

- RLC filter

- low-pass filter with the appropriate bandwidth

23. The processing gain of direct sequence Spread Spectrum System is

- independent of bandwidth of data signal

- independant of bandwidth of spreaded signal.

- related to modulation index

24.  The 8085 microprocessor has how many address lines?

A.  8

B. 12

C. 16

D.  20

25. The instruction MOV A, B performs:

A. Moves A to B

B.  Moves B to A

C. Adds A and B

D. Subtracts B from A