|  | # FACULTY OF _____________________________ # DEPARTMENT OF ___________________________ |
| --- | --- |

B

| Name | :___________________________ | Department | : ________________________ |
| --- | --- | --- | --- |
| Application No. | :___________________________ | Date | : 16/05/2026 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to Candidates - This question paper consists of Multiple Choice Questions (MCQs) only. - Each question has four options (a, b, c, and d), out of which only one option is correct. - Each correct answer carries one mark. There is no negative marking for incorrect answers. - Candidates must mark their answers only on the OMR sheet provided. - The use of calculators, mobile phones, smartwatches, or any electronic devices is strictly prohibited inside the examination hall. - Reference materials, notes, books, or any printed resources are not permitted during the examination. - Use only a black or blue ballpoint pen to mark the answers on the OMR sheet. - Rough work should be done only in the space provided in the question paper booklet. - Before starting the examination, ensure that all required details (name, application number, etc.) are correctly filled in on the OMR sheet. - Do not fold, damage, or mark the OMR sheet unnecessarily, as it will be scanned for evaluation.

Subject-Specific Questions

(25×1=25)

### 1. The antenna current of an AM broadcast transmitter ,modulated to a depth of 40 percent by an audio sine wave is 11 A.It increases to 12 A as a result of simultaneous modulation by another audio sine wave. What is the modulation index due to this second wave.

### A. 0.643

### B. 0.1
C. 0.2
D. 10

### Answer: A

2. A broadcasting station has transmitter   power of 10.125 kW when it is modulated by sine wave with modulation index 0.5.If another sine wave is simultaneously transmitted with modulation index 0.4,what is the effective modulation index.

- A. 0.64

- B.  1

- C.  1.5

- D. 2.9

Answer: A

3. A SSB transmitter radiates 0.5 kW when the modulation percentage is 60 %.

How much of carrier power is required if we want to transmit the same message by an DSB-AM transmitter.

- A. 6.56 kW

- B. 7.85 kW

- C. 10.6 kW

- D. 5.99 kW

Answer: A

4. An AM news broadcasting station uses a carrier signal of 1 MHz.If the speech signal has frequency components up to 8 kHz,What is the lowest frequency in AM signal.

- A. 0.844 MHz.

- B. 0.922 MHz

- C. 1.87 MHz

- D.  2.78 MHz

- Answer: B

5.  In an FM system, the audio frequency is 500 Hz and frequency deviation is 4.8 kHz.What is the modulation index?

- A. 10.2

- B. 1.9

- C. 9.6

- D. 12

Answer: C.

### 6. A system described by  is….

### A. Non-linear and time-invariant
B. Linear and causal
C. Non-causal and stable
D. Time-variant

### Answer: B

7. Thevenin’s theorem replaces a network by:

A.Voltage source in shunt with resistance

B. Voltage source in series with resistance

C.Current source in series with resistance

D. Voltage source in shunt with current source

Answer: B.

8. In a purely resistive AC circuit, power factor is:

- A. 10

- B. 0

- C. 1

- D. 12

Answer: C.

9. In binary PSK, the bit rate equals the

A.0.5 x Baud rate

B.Baud Rate X 2

C.Baud rate

D.Baud Rate divide by 2.

Answer : C

10. In a BJT, if base current is  collector current is…..

A. 10 µA
B. 100 µA
C. 1 mA
D. 10 mA

Answer: C

### 11. The function of a PLL (Phase-Locked Loop) in communication is to….

### A. Increase transmission power
B. Generate harmonics
C. Track and lock to input frequency
D. compress bandwidth

### Answer: C

### 12. Which property must a system have to allow convolution in time domain to be represented as multiplication in frequency domain?

### A. Time invariance
B. Linearity
C. Causality
D. Both A and B

### Answer: D

### 13. The cut-off frequency in an RC low-pass filter is defined as the frequency where the output voltage is….

### A. Equal to the input voltage
B. Half of the input voltage
C. 70.7% of the input voltage
D. Zero

### Answer: C

### 14. In a sinusoidal AM wave, if the modulation index is 1, then the total power is….

A. Equal to carrier power
B. Twice the carrier power
C. 1.5 times the carrier power
D. Three times the carrier power

Answer: C

15. The voltage gain of an inverting amplifier is (–100). If input voltage is 20 mV, output voltage is:

- A. 1 V

- B. 100 mV

- C. 0.7 V

- D. -2 V

Answer: D

16. The Boolean expression AB + A'B simplifies to:

- A. AB + A'B

- B. B

- C. AB

- D. A

Answer: B

17. The number of flip-flops required to design a mod-16 counter is:

- A.  10

- B.  4

- C.  1

- D. 2

Answer: B

18. The spectral efficiency of 16-QAM is higher than BPSK because…..

A. It has lower BER 
B. It transmits more bits per symbol
C. It uses lower bandwidth
D. It uses phase shift  only

Answer: B

### 19. Which of the following is not an advantage of CMOS over TTL?

### A. Low static power consumption
B. High packing density
C. High noise margin
D. High power dissipation

### Answer: D

### 20. A unity feedback system has open-loop transfer function  ​. The value of K for critical damping is…

### A. 10
B. 40
C. 8
D. 20

### Answer: C

### 21. In an 8-bit successive approximation ADC with a clock of 1 MHz, the total conversion time is approximately…

### A) 8 μs
B) 1 μs
C) 0.125 μs
D) 64 μs

### Answer: A

### 22. For a Butterworth low-pass filter of order N, the magnitude response is maximally flat at:

### A) Stopband only
B) Cut-off frequency
C) Origin
D) All frequencies

### Answer: B

### 23. A signal occupies 10 kHz bandwidth. The minimum sampling rate to ensure perfect reconstruction is….

### A) 10 kHz
B) 50 kHz
C) 15 kHz
D) 20 kHz

### Answer: D

### 24. A signal  is sampled at 200 Hz. The resulting sampled signal will exhibit….

### A) Aliasing of both components
B) No aliasing
C) Aliasing of one component
D) Cannot be determined without Nyquist rate

### Answer: C

### 25. The phase margin of a system is measured at the frequency where…?

### A. Gain is 0 dB
B. Phase is -180°
C. Phase is 0°
D. Gain is maximum

Answer .A

Answer Key

B

| Question No. | Correct Option | Question No. | Correct Option |
| --- | --- | --- | --- |
| 1 | A | 26 |  |
| 2 | A | 27 |  |
| 3 | A | 28 |  |
| 4 | B | 29 |  |
| 5 | C | 30 |  |
| 6 | B | 31 |  |
| 7 | B | 32 |  |
| 8 | C | 33 |  |
| 9 | C | 34 |  |
| 10 | C | 35 |  |
| 11 | C | 36 |  |
| 12 | D | 37 |  |
| 13 | C | 38 |  |
| 14 | C | 39 |  |
| 15 | D | 40 |  |
| 16 | B | 41 |  |
| 17 | B | 42 |  |
| 18 | B | 43 |  |
| 19 | D | 44 |  |
| 20 | C | 45 |  |
| 21 | A | 46 |  |
| 22 | B | 47 |  |
| 23 | D | 48 |  |
| 24 | C | 49 |  |
| 25 | A | 50 |  |

### Note to Question Paper Setters - Format Requirements - Font: Times New Roman - Font Size: 12 - Equations - All equations must be created using MathType or the Microsoft Equation Editor. - Images of equations should not be inserted. - Figures and Images - Any figures or images included must be clear, high-resolution, and suitable for printed format. - Answer Indication - The correct answer should be clearly bolded in the submitted question paper template. - Examination Rules - The use of calculators or reference materials is not permitted during the examination. - Question Set Preparation - Questions must not be repeated across Set A and Set B. - Questions with similar meaning or concept duplication should be avoided across the sets to ensure a diverse assessment of candidates' knowledge and critical thinking skills. - Standard of Questions - The questions should be appropriate for a PhD entrance examination and cover both: - Research Methodology - Subject-specific advanced concepts - The answer keys must be provided in the template. - The name of the institution and the department must be clearly mentioned at the top of the template.