|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING |
| --- | --- |

B

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : EEE |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

- In a complex resistive network powered by a 10V source, two nodes A and B are part of the circuit. With all components connected, their potentials are VA = 6V and VB = 4V relative to ground. If the resistor RAB between A and B is now removed, creating an open circuit, what is the new voltage VAB = VA - VB measured across the open terminals?

- 2 V

- Cannot be determined without the full circuit diagram

- 0 V

- 10 V

- An ideal 10 V voltage source is connected to two 10 Ω resistors in series, and the total power dissipated by the circuit is 𝑃total. One of the 10 Ω resistors is then replaced by an open circuit. What will be the new total power dissipated 𝑃new, in the entire circuit?

- 𝑃new = 2 X 𝑃total

- 𝑃new = 𝑃total / 2

- 0

- 𝑃new = 𝑃total

- An inductor carries steady current I0. At t = 0, it is disconnected. What occurs ideally at t = 0+?

  - Inductor current instantaneously drops to zero

  - Inductor generates an infinite voltage spike

  - Both current and voltage remain finite

  - Energy stored in inductor is instantly lost

- A resistive 3-phase load is connected in star and consumes 3 kW at line voltage VL. If the same resistors are reconnected in delta to the same supply, total power becomes

  - 9 kW

  - 6 kW

  - 4.5 kW

  - 3 kW

- In a series RLC circuit, the phase angle of the total impedance Z at the lower half-power frequency    f1 < fo   is

  - +90o

  - -90o

  - +45o

  - -45o

- A DC shunt generator fails to build up voltage. You have confirmed residual magnetism is present and the field circuit is not open. Which of the following is the most likely remaining cause?

- The armature resistance is too high

- The field resistance is less than the critical field resistance

- The direction of rotation is reversed

- The prime mover speed is above the critical speed

- The steel core of a transformer is constructed from thin, insulated laminations. If these laminations were not insulated from each other, what would be the immediate consequence upon energization?

  - The transformer would fail to step up/down the voltage

  - The core would act as a solid block, leading to massive eddy currents and rapid overheating

  - The hysteresis loss would increase dramatically, but eddy current loss would be unaffected

  - The magnetizing current would drop to near zero

- When the supply voltage of an induction motor is reduced to half while keeping the frequency constant, how does the maximum torque change?

- It remains the same

- It becomes half of its original value

- It becomes one-fourth of its original value

- It becomes double the original value

- In a synchronous motor operating at unity power factor, if the load torque increases, then to maintain synchronism

- The torque angle increases

- The stator frequency must increase

- The rotor speed decreases slightly

- The excitation current must increase

- When a magnetic system has both electrical and mechanical inputs, energy conversion occurs if

- Magnetic field intensity varies with time

- There is relative motion between field and conductors

- Magnetic saturation occurs

- Both flux linkages and mechanical displacement vary simultaneously

- In a boost converter, when the load decreases suddenly, maintaining continuous conduction mode becomes difficult because

- The diode’s reverse recovery charge becomes dominant, reducing current continuity

- The inductor current decays to zero before the next switching cycle begins

- The reduced load current forces the capacitor to conduct reverse current continuously

- The switch remains off for a longer duration, keeping the inductor current constant

- In the context of triggering, the V-I characteristic curve of the gate-cathode junction of an SCR is analogous to

- A Zener diode operating in its breakdown region

- A high-impedance reverse-biased junction

- A standard forward-biased P-N junction

- A low-barrier-potential Schottky contact

- A single-phase fully controlled converter operates in inversion mode when

- The firing angle is less than 90° and current flows from AC to DC side

- The firing angle is close to 0°, resulting in maximum average DC voltage

- The firing angle is exactly 90°, provided the load current is continuous and source voltage is sinusoidal

- The firing angle is greater than 90°, causing average DC voltage to become negative

- In a three-phase voltage source inverter operating with 120° conduction mode, at any instant of time

- Only one switch conducts, corresponding to the phase with maximum voltage

- Two switches conduct simultaneously, each from a different leg

- One switch in each leg conducts, forming a current path for all three phases

- All six switches conduct alternately to maintain symmetry

- In a single-phase half-wave AC voltage controller, the RMS output voltage becomes zero when the firing angle is

- 0o

- 45o

- 90o

- 180o

- What is the fundamental purpose of transposing the conductors in a double-circuit, three-phase transmission line?

- Balance the inductive effects between phases

- Reduce mechanical vibrations of the line

- Make the resistance of all conductors equal

- Minimize line-to-line capacitance to achieve symmetrical charging current distribution

- The voltage drop in a radial distribution feeder is a function of its impedance and the load it serves. Which of these factors has the most direct and proportional impact on the magnitude of the voltage drop?

- Line reactance

- Load power factor

- Conductor resistance

- Load current

- The Ferranti effect in long transmission lines is due to

- Low line resistance and high load current

- Voltage drop across the line reactance under heavy load

- Charging current caused by line capacitance at light load

- Resonant effect involving the line's series inductance and series capacitance

- The rupturing capacity of a fuse is a critical safety specification. This rating defines the fuse's ability to

- Carry its nominal current indefinitely without overheating

- Withstand the maximum current it can interrupt safely without damage

- Melt at the minimum current specified by its time-current curve to ensure coordination with other devices

- Limit the total let-through energy to protect downstream equipment from thermal damage

- A single-core cable's capacitance per unit length is modelled as a coaxial capacitor. This capacitance value is a function of which two properties?

- The insulation's relative permittivity and the sizes of the conductor and sheath

- The conductor's resistance and the thickness of the insulation

- The insulation's breakdown voltage and the total length of the cable

- The sheath's material and the conductor's area

- An RTD (Resistance Temperature Detector) operates based on which of the following fundamental principles?

- Resistance increases linearly with temperature

- Resistance decreases linearly with temperature

- Resistance increases nonlinearly with temperature

- Resistance decreases nonlinearly with temperature

- Adding a pole at the origin to the open-loop transfer function of a stable system generally

  - Improves both steady-state and transient performance

  - Eliminates steady-state error and improves transient speed

  - Eliminates steady-state error and worsen transient response

  - Has no effect on steady-state accuracy or transient response

- In the frequency domain, what is the primary contribution of the integral (I) term to the open-loop transfer function?

- Increasing the phase margin

- Making the system more robust but slower

- Increasing high-frequency gain and improving transient response

- Ensuring zero steady-state error to a step input

- For a linear time-invariant system to be classified as asymptotically stable, where must all poles of its closed-loop transfer function be located in the complex s-plane?

- All poles must lie on or to the left of the jω axis

- All poles must lie strictly to the left of the jω axis

- All poles must be simple (non-repeated) and lie on the jω axis

- All poles must have zero-valued real parts

- A unity feedback system is characterized by a Type 1 open-loop transfer function G(s). What is the value of this system's position error constant (Kp), and what is the resulting steady-state error (ess) for a unit step input?

- Kp = ∞  and  ess = 0

- Kp = 0  and  ess = 1

- Kp = 0  and  ess = ∞

- Kp = 1  and  ess = 0