|  | # FACULTY OF ENGINEERING AND TECHNOLOGY # DEPARTMENT OF CIVIL ENGINEERING |
| --- | --- |

A

Ph.D. Entrance Examination – January 2026

MCQ Type Question Paper

| Name | :_______________________________ | Department | : Civil Engineering |
| --- | --- | --- | --- |
| Application No. | :_______________________________ | Date | : 10/11/2025 |
|  |  | Duration | : 2 hrs |
|  |  | Maximum Marks | : 50×1=50 |

Instructions to candidates: - All questions are Multiple Choice Questions (MCQs). - Each question has four options, of which only one is correct. - Each correct answer carries one mark. - There is no negative marking for incorrect answers. - The use of calculators, mobile phones, smartwatches, or any reference material is strictly prohibited during the examination. - Use only a black or blue ballpoint pen to mark your answers in the OMR sheet. - Perform rough work only in the space provided in the question paper booklet. - Ensure that all required details (such as Name, Registration Number, etc.) are filled in correctly on the OMR sheet before starting the exam.

Section B: Subject-Specific Core Topics (25 Questions | 1 Mark Each)

26. The ratio of inertia force to viscous force is known as:
A. Froude number
B. Reynolds number
C. Weber number
D.  Mach number

27. If a 2-legged 8 mm diameter HYSD bar is used as shear reinforcement for a beam of width 230 mm and effective depth 300 mm. what is the nearest magnitude of the spacing of minimum shear reinforcement?

- 420 mm

- 350 mm

- 390 mm

- 320 mm.

28. A welded plate girder, consisting of two flange plates of 350 mm x 16 mm each and a web plate of 1000 mm × 6 mm, requires

  - no stiffeners

  - horizontal stiffeners

  - intermediate vertical stiffeners

  - vertical and horizontal stiffeners

29. Steel of yield strength 400 MPa has been used in a structure. What is the value of the maximum allowable tensile strength?

  - 120 MPa

  - 240 MPa

  - 200 MPa

  - 96 MPa

30. An undrained triaxial compression test is carried out on saturated clay sample under a cell pressure of, 100 kN/m2. The sample failed at a deviator stress of 200 kN/m2. The cohesion of the given sample of clay is

- 100 kN/m2

- 150 kN/m2

- 200 kN/m2

- 50 kN/m2

31. A 4-hour direct runoff hydrograph of a catchment is triangular in shape with a time base of 100 hours and peak flow of 50 m3/sec. The catchment area is 360 km2.The peak low of this catchment area for a 4-hour unit hydrograph is

- 10m3/sec

- 20m3/sec

- 25m3/sec

- 50m3/sec

32. If the undrained cohesion of saturated clay is 50 kPa, its shear strength is equal to

- 25 kPa

- 50 kPa

- 100 kPa

- 200 kPa

33. A fixed beam AB is subjected to a triangular load varying from zero at end A to w per unit length at end B. The ration of fixed end moment at B to A will be

A. 1/2

B. 1/3

C. 2/3

D. 3/2

34. A built-up double laced column is 8 m long and one end of the column is fixed and other end is free. The effective length of the laced column will

  - 17.6 m

  - 16 m

  - 16.8 m

  - 8.8m

35. The continuity equation expresses the law of:

- Momentum

- Energy conservation

- Mass conservation

- Inertia

36. The radius of a horizontal circular curve is 480 m, and the design speed is 70 km/h. What will be the equilibrium superelevation for equal pressure on the inner and outer wheels?

- 5%

- 6%

- 7%

- 8%

37. The fixed support in a real beam becomes in the conjugate beam a

- roller support

- fixed support

- hinged support

- free end

38. Prestressed concrete is more desirable in case of

- cylindrical pipe subjected to internal fluid pressure

- cylindrical pipe subjected to external fluid pressure

- cylindrical pipe subjected to equal internal and external fluid pressure

- cylindrical pipe subjected to end pressures.

39. Sewage treatment in an oxidation pond is accomplished primarily by

- alga-bacteria symbiosis

- algal photosynthesis only

- bacteria oxidation only

- chemical only

40. Standard 5-day BOD of a waste water sample is nearly x% of the ultimate BOD, where x is

- 48

- 58

- 68

- 78

41. What is the imperfection factor for lateral torsional buckling of beams for welded steel sections (a)

- 0.65

- 0.21

- 0.45

- 0.49

42. The torsional stiffness of a member can be defined as

  - Torque for unit moment

  - Torque for unit twist

  - Moment for unit twist

  - Torsion for unit twist

43. A 30 m metric chain is found to be 0.1-m too short throughout the measurement. If the distance measured is recorded as 300 m, then the actual distance will be

- 300.1 m

- 301.0 m

- 299.0m

- 310.0m

44. Partially safety factor for concrete and steel in LSM of design of RC structures assumed as

- 1.35 & 1.20 respectively

- 1.08 & 1.25

- 1.24 & 1.25

- 1.5 & 1.15

45. The effective size of sand particles used in slow sand filters is

  - 0.25 to 0.35 mm

  - 0.60 to 1.00mm

  - 0.35 to 0.60 mm

  - 1.00 to 1.80 mm

46. Compute the safe bearing capacity of a continuous footing 1.8 m wide, and located at a depth of  1.2 m below ground level in a soil with unit weight γ = 20 kN/m3, c = 20 kN/m2, and φ = 20°. Assume a factor of safety of 2.5. Terzaghi’s bearing capacity factors for φ = 20° are Nc = 17.7, Nq = 7.4, and Nγ = 5.0, what is the permissible load per metre run of the footing?

- 239 kN/m

- 597 kN/m

- 263 kN/m

- 473 kN/m

47. The Kor depth for rice is 190 mm and Kor period is 14 days. The outlet factor for this will be

  - 637 hectares/m3/sec

  - 837 hectares/m3/sec

  - 972 hectares/m3/sec

  - 1172hectares/m3/sec

48. A load of 4000 kN is uniformly distributed over an area of 3 m x 3 m. Average vertical stress at a depth of 2 m below will be (assuming 2 vertical: 1 horizontal distribution)

  - 250 kN/m

  - 160 kN/m

  - 200 kN/m

  - 220kN/m

49. A soil has liquid limit=32, plastic limit=18, shrinkage limit = 8 and natural moisture content=22%. What will be its liquidity index and plasticity index?

- 0.67 and 15

- 0.285 and 14

- 0.67 and 25

- 0.33 and 20

50. Coefficient of velocity of venturimeter

  - is independent of Reynolds number

  - decreases with higher Reynolds number

  - is equal to the coefficient of discharge of venturimeter

  - none of the above